### CENWP-OP-B                                                                                               29 November 1999

MEMORANDUM FOR CENWP-OP, ATTN:  Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 47 of 1999, which covers the period 21 through 27 November.

- OPERATION SUMMARY.

1.1. Daily average river flow ranged from 156.6 to 223.7 kcfs.  Daily average tailwater elevation for the project ranged from 14.6’ to 20.8’ msl.  River temperature ranged from 52 to 53° F.  Spill bays 1 and 18, which supply attraction flow for the Cascades Island and B-branch fish ladders respectively, remain open 4”.

- First powerhouse Unit 4 remains out of service for rehab, unit 10 for stator rewind, unit 11 for four-year overhaul, and unit 16 for stator re-wedge.  Unit 15 was out of service from 0506-0848 on 26 November due to a failed STS.

1.3. The first powerhouse ice and trash sluiceway remained open for debris removal this reporting week.  The second powerhouse ice and trash sluiceway remained closed.

1.4. Second powerhouse fish units remained in service this reporting week except when placed on standby from approximately 0000-0300 to float trash if drawdown exceeds 1.0’.

- ACTIVITIES.

2.1. STS Inspections.  There were no STS’s inspected this reporting week.

2.2. VBS Inspections.  There were no VBS inspections this reporting week.

2.3. AWS Closures.  There were no AWS closures this reporting week.

2.4. Dewaterings and Fish Salvage.  The PH2 DSM was diverted to ERC release and the outfall dewatered on 22 November this reporting week to replace the sills on the upper and lower switch gates.  Project biologists recovered 2 adult hatchery steelhead from the bypass flume.  The DSM and outfall were restored to normal operation at 1811 on 23 November.

- RESEARCH.

3.1. Normandeau MGR Testing.  Normandeau and Mid-Columbia Consultants continue biological MGR testing at units 5 and 6.

3.2. WDFW Fish Counting.  WDFW continues 24-h/d video counting at Bradford Island and Washington shore.

4.  FISHWAYS.  Project biologists inspected on 22, 23 and 24 November this reporting week.

4.1. Fish Passage Plan Observations.

| DATE | LOCATion | FPP VIOLATION | CAUSE | RESPONSE |
| --- | --- | --- | --- | --- |
| All Week | A-branch diffuser FG3-4. | Diffuser should be open at 14.4’ tailwater elevation and is stuck closed due to mechanical failure. | Diffuser gate is separated from its shaft and will not be repaired until the winter maintenance season. | Other diffusers are being operated in place of FG3-4 depending on tailwater elevations. |
| All Week | A-branch diffuser FG3-6. | Diffuser should be open at 20.4’ tailwater elevation and is stuck closed due to mechanical failure. | Diffuser gate is separated from its shaft and will not be repaired until the winter maintenance season. | Other diffusers are being operated in place of FG3-4 depending on tailwater elevations. |
| All Week | PH1 orifice gate 21. | Gate should be closed according to tailwater elevations. | Panel is missing, allowing water to pass freely when gate is closed. | Sluice gate has been closed and orifice gate will be repaired this winter maintenance season. |
| All week | PH1 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps. | Unknown. Meter is working intermittently. | No action taken. |
| 11/19 | PH2 collection channel electronic velocity meter. | Meter was not working and should register 1.5-4.0 fps. | Unknown. Meter is working intermittently. | No action taken. |

4.2. Sensor Calibration.  Water level sensors were checked for calibration on 24 November.

4.3. Avian Predation Status.  There are currently three avian lines down at PH1.

4.4. Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- CONSTRUCTION UPDATE.

5.1. Construction continues on the new Juvenile Monitoring Facility.

DEBORAH L. CHENOWETH

# Operations Project Manager