### CENWP-OP-B                                                                                                21 December 1999

MEMORANDUM FOR CENWP-OP, ATTN:  Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 50 of 1999, which covers the period 12 through 18 December.

- OPERATION SUMMARY.

1.1. Daily average river flow ranged from 187.1 to 236.8 kcfs.  Daily average tailwater elevation for the project ranged from 17.2’ to 20.7’ msl.  River temperature ranged from 47 to 48° F.  Spill bay 1, which supplies attraction flow for the Cascades Island ladder remained on sill as of 8 December.  Spill bay 18, which supplies attraction flow for the B-branch ladder remained open 4”.

- Unit 10 remained out of service for stator rewind.  Unit 4 returned to service following commissioning testing at 0737 on 15 December.  Unit 11 returned to service following four-year overhaul at 1030 on 14 December.  Units 1, 2 and 3 were out of service from approximately 0700-1700 on 13 through 18 December for divers to install surface collector equipment.

1.3. The first powerhouse ice and trash sluiceway remained open for debris removal this reporting week.  The second powerhouse ice and trash sluiceway was opened for debris removal from 1555-1630 and 1248-1415 on 15 and 17 December, respectively.

1.4. Second powerhouse fish units remained out of service for winter maintenance.

- ACTIVITIES.

2.1. STS Removal.  PH1 and PH2 STS’s were removed on 16 through 18 December.

2.3. AWS Closures.  FV1-1 was closed for cleaning from 1610-1630 on 14 December.

2.4. Dewaterings and Fish Salvage.  Unit 11 tailogs were removed on 13 December in preparation for returning the unit to service.  Project biologists recovered 13 sculpins, one peamouth chub, one catfish, and one sturgeon.

- RESEARCH.

3.1. Normandeau MGR Testing.  Normandeau and Mid-Columbia Consultants continued biological MGR testing at units 5 and 6.

3.2. WDFW Fish Counting.  WDFW continued 24-h/d video counting at Bradford Island and Cascades Island.

4.  FISHWAYS.  Project biologists inspected on 14 and 16 December this reporting week.

4.1. Fish Passage Plan Observations.

| DATE | LOCATion | FPP VIOLATION | CAUSE | RESPONSE |
| --- | --- | --- | --- | --- |
| One day this reporting week. |  | Project biologists did not complete 3 fishway inspections as required per the fish passage plan. | Additional activities. | Operators continue to inspect fishways daily. |
| All Week | A-branch diffuser FG3-4. | Diffuser should be open at 14.4’ tailwater elevation and is stuck closed due to mechanical failure. | Diffuser gate is separated from its shaft and will not be repaired until the winter maintenance season. | Collection channel is maintaining criteria despite diffuser being out of service. |
| All Week | A-branch diffuser FG3-5. | Diffuser should be open at 17.4’ tailwater elevation and is stuck closed due to mechanical failure. | Diffuser gate is separated from its shaft and will not be repaired until the winter maintenance season. | Collection channel is maintaining criteria despite diffuser being out of service. |
| All Week | A-branch diffuser FG3-6. | Diffuser should be open at 20.4’ tailwater elevation and is stuck closed due to mechanical failure. | Diffuser gate is separated from its shaft and will not be repaired until the winter maintenance season. | Collection channel is maintaining criteria despite diffuser being out of service. |

4.2. Sensor Calibration.  Water level sensors were checked for calibration on 16 December.

4.3. Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

5.  OTHER NOTABLE ITEMS.

5.1. The PH2 DSM and outfall flume will be dewatered on 20 December.

5.2. The Cascades Island ladder will be dewatered on 21 December.

5.3. The PH2 outfall remained at high outfall release.

- CONSTRUCTION UPDATE.

6.1. Construction continues on the new Juvenile Monitoring Facility.

DEBORAH L. CHENOWETH

# Operations Project Manager