### CENWP-OP-B                                                                                                28 December 1999

MEMORANDUM FOR CENWP-OP, ATTN:  Gary Johnson

SUBJECT: Bonneville Lock and Dam, Fishway and Fish Activities for week 51 of 1999, which covers the period 19 through 25 December.

- OPERATION SUMMARY.

1.1. Daily average river flow ranged from 213.7 to 239.7 kcfs.  Daily average tailwater elevation for the project ranged from 17.9’ to 20.7’ msl.  River temperature ranged from 46 to 47° F.  Spill bay 1, which supplies attraction flow for the Cascades Island ladder remained on sill as of 8 December.  Spill bay 18, which supplies attraction flow for the B-branch ladder remained open 4”.

- Unit 10 remained out of service for stator rewind.  Units 1, 2 and 3 were out of service from approximately 0700-1700 on 20 through 23 December for divers to install surface collector equipment.

1.3. The first powerhouse ice and trash sluiceway remained open for debris removal this reporting week.  The second powerhouse ice and trash sluiceway was opened for debris removal from 1230-1506 on 20 December.

1.4. Second powerhouse fish units remained out of service for winter maintenance.

- ACTIVITIES.

2.1. AWS Closures.  FV5-3, 5-4, and 5-9 were closed on 19 December in preparation for the Cascades Island dewatering.  FV1-1 was closed for cleaning from 1609-1630 on 20 December and 1545-1555 on 22 December.

2.2. Dewaterings and Fish Salvage.

- The PH2 DSM and outfall were dewatered on 20 December for maintenance.  Project biologists recovered no fish from the DSM channel and one dead adult steelhead was recovered from the outfall flume due to a failure with the flume flushing water.

- Cascades Island ladder was dewatered on 21 December for winter maintenance.  Project biologists recovered 2 juvenile salmonids, one adult wild steelhead, and one whitefish.

- RESEARCH.

3.1. Normandeau MGR Testing.  Normandeau and Mid-Columbia Consultants continued biological MGR testing at units 5 and 6 on 19 through 22 December this reporting week.  They will resume testing on 5 January.

3.2. WDFW Fish Counting.  WDFW continued 24-h/d video counting at Bradford Island.  Video counting was discontinued at Cascades Island on 20 December in preparation for dewatering.

4.  FISHWAYS.  Project biologists inspected on 20, 21 and 22 December this reporting week.

4.1. Fish Passage Plan Observations.

| DATE | LOCATion | FPP VIOLATION | CAUSE | RESPONSE |
| --- | --- | --- | --- | --- |
| All Week | A-branch diffuser FG3-4. | Diffuser should be open at 14.4’ tailwater elevation and is stuck closed due to mechanical failure. | Diffuser gate is separated from its shaft and will not be repaired until the winter maintenance season. | Collection channel is maintaining criteria despite diffuser being out of service. |
| All Week | A-branch diffuser FG3-5. | Diffuser should be open at 17.4’ tailwater elevation and is stuck closed due to mechanical failure. | Diffuser gate is separated from its shaft and will not be repaired until the winter maintenance season. | Collection channel is maintaining criteria despite diffuser being out of service. |
| All Week | A-branch diffuser FG3-6. | Diffuser should be open at 20.4’ tailwater elevation and is stuck closed due to mechanical failure. | Diffuser gate is separated from its shaft and will not be repaired until the winter maintenance season. | Collection channel is maintaining criteria despite diffuser being out of service. |
| All Week. | PH1 collection channel electronic velocity meter. | Meter registered <1.5 fps and should register 1.5-4.0 fps. |  | No action taken. |

4.2. Sensor Calibration.  Water level sensors were checked for calibration on 22 December.

4.3. Zebra Mussel Monitoring.  The zebra mussel monitoring station was inspected and showed no signs of colonization this reporting week.

- CONSTRUCTION UPDATE.

5.1. Construction continues on the new Juvenile Monitoring Facility.

DEBORAH L. CHENOWETH

# Operations Project Manager