U.S. ARMY CORPS OF ENGINEERS

WALLA WALLA DISTRICT

#### FISH FACILITIES WEEKLY REPORT

#26-2008

# Project:  McNary

# Biologists:  Brad Eby and Bobby Johnson

Dates:  August 22 - 28, 2008

## Turbine Operation

McNary had 12 to13 turbine units available for power generation this week.  Outages included turbine unit 5 (9-year overhaul, expected to return to service in mid-September), turbine unit 13 (returned to service from annual maintenance on August 22), turbine unit 3 (annual maintenance from August 25 to 28), and turbine unit 2 (governor work, August 27 to 28).  Due to low river flows and spill program requirements, many turbine units were in standby status for extended periods.  No turbine units operated outside the 1% criteria this week.

## Adult Fish Passage Facilities

On August 23, 25 and 28, McNary fisheries biologists performed measured inspections of the adult fishways.  On August 28, the Oregon ladder entrances were inspected twice, during and after the outage of fish pump 2 described below.  Adult fish counting and temperature monitoring of the ladders’ exits continued.

### Fish Ladders:  During measured inspections, both ladders met all Fish Passage Plan criteria except on August 25 when the Oregon ladder head over weir measured 0.9 feet.  The Oregon ladder’s north traveling screen remains out of service and project personnel continued to work towards resolving this issue.  The south traveling screen continued to operate well.  Washington exit weir 340 triggered an alarm on one occasion and was reset.  Otherwise, regulating weirs operated satisfactory this week.  Both Washington and Oregon ladder exit set points were checked twice this week, with only one slight change noted at the Oregon ladder exit.

Fishway Entrances and Collection Channel:  All Washington ladder criteria points were met except on August 23 when W3 measured 7.8 feet.  Some Oregon fishway entrances were slightly below weir depth criteria (9.0’) this week including NFEW2 on August 25 and 28 (both 8.9’), NFEW3 on August 28 (8.9’), and SFEW1 and SFEW2 (both 8.7’ to 8.8’ throughout the week). The south pool differential measured within criteria when two fish pumps were in use.  The channel velocity averaged 1.3 feet per second.

Auxiliary Water Supply System:  Fish pump 3 remains out of service due to an oil leak.  Pumps 1 and 2 ran all week with blade angles of 30 degrees except when pump 2 was out of service.  Pump 2 was out of service on August 28 from 1210 to 1620 hours in support of pump 3 intake bulkhead removal.  Divers will be needed to inspect, clean and set the bottom bulkhead before this pump can be dewatered.  This will likely take place at a later time, probably during the normal winter outage.  The juvenile facility continued to supply the usual 450 cfs to the north powerhouse pool.  The Wasco County PUD unit had no interruptions in service this week.

#### Juvenile Fish Passage Facility

Collection for truck transport continues.  On August 24 from 0700 to 1242 hours, primary bypass took place to allow sample gate repairs.  The B side sample gate actuator was removed, repaired and reinstalled.  It is estimated the actuator failed sometime between the hours of 0300 and 0600, based on hourly fish count numbers.  For the report week, 30,200 smolts were collected and trucked down river.

Forebay Debris/Gatewell Debris/Oil:  :  Forebay debris was light to very light as wind and flow changes have dispersed much of the floating debris.  No trash racks were cleaned and differentials continued to be monitored.  No problems were observed within the gatewell slots.

ESBSs/VBSs:  ESBSs are currently deployed in all turbine unit slots except for those in slots 5B and 5C.  One screen is still undergoing maintenance and the other is being kept in reserve as a spare as turbine unit 5 is out of service.  Fish screens in slots 4B and 11C slots remain in transducer bypass mode.  On August 25, the technical staff returned the screen in slot 1A to automatic operation from bypass mode.  ESBS camera inspections will resume once a new camera cable is ordered and installed.  Two high VBS differentials were recorded this week.  Affected screens were cleaned on August 25 and 26 along with 11 other VBSs.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:  During the week, 40 to 41 orifices were opened depending on forebay elevation.  All turbine unit 5 orifices were closed as the turbine unit and associated ESBSs were out of service.  Additional orifices were opened in adjacent turbine units to maintain proper collection channel elevation.  All systems functioned well and orifice lights were serviced this week.  The side and rectangular cleaning brush cycle times were respectively set between 90 to 180 minutes and between 75 to 120 minutes depending on expected debris load from storms or turbine unit operational changes.  Although no heavy debris was observed during this report period, milfoil continues to be a problem.

Transportation Facility:  Collection for truck transport continued as barge operations ended August 16.  PIT tag detection took place in the full flow pipe and throughout the facility with smolt monitoring occurring daily.

All systems functioned well.  The primary (A and B) PIT tag slide gates remained on, and the secondary (C and D) PIT/bypass gates were closed (off for duration of transport operations).  Balloon test of the PIT tag release lines revealed no problems (Project personnel release balloons filled with air and water into the PIT tag lines – if intact balloons are observed exiting the outfall, there are no blockages).  The sample gates remained on with no adjustments being made except for the B side sample gate failure as mentioned above.  When sampling at rates of 20 percent or higher, PIT tag diversion gates are set to override the sample gates.  On August 22 at 0700 hours, the PIT tag override was inadvertently left on when the sample rate was changed to 10 percent.  This configuration was not discovered until 0738 hours on August 23.  With so few fish present in the river, the impact on sample quality was likely very minimal.

Transport Summary:  Truck loading occurred on August 23, 25 and 27.  At the new truck release site at Bonneville Dam, some issues with flush water and the release line continued to be addressed.  On August 25, the driver noted a few dried fish moralities in the release line before unloading the tanker.  He could not determine how many smolts had been lost.  On August 27, the project biologist went to the site to observe the operation.  Adjustments are being made in operating procedures to improve release conditions.

River Conditions

River conditions during the week are outlined in Table 1 as provided by the smolt monitoring staff, with the data day running from 0700 hours to 0700 hours.  The 60/40 spill program continues until 0001 hours on September 1.  The 60/40 spill program continues, with adjustments at times to assist  river traffic entering and exiting the navigation lock  Spill percentage was reduced August 26 and 27 due to low river flows and the need to meet minimum power generating loads (minimum of 50 kcfs needed to support powerhouse load).  In addition, spill gates were serviced this week.  On September 1, smolt monitoring staff will remove water temperature probes in various locations throughout the project.  These probes have been in place since June 14.  Results will be published in a separate report.

Table 1.  River conditions at McNary Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity* (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 145.9 | 120.8 | 86.4 | 53.0 | 70.1 | 68.1 | 6.0 | 6.0 |

*Morning wet lab temperature.  Water clarity provided by control room.

Other

Bird counts continue with the tailrace locations being checked twice a day and the forebay locations being checked once per day.  Bird numbers on project appear to be fluctuating significantly and probably are affected by changes in out migrating juvenile shad numbers..  The high tailrace count was approximately 200 gulls, 37 terns and 42 cormorants, and an occasional osprey and pelican being noted.  No birds were seen near the bypass outfalls.  An occasional grebe or cormorant was noted in the forebay.  The water cannon remains in service as PIT tagged fish are exiting one of the outfalls.

On August 26, the first bull trout seen in many years was encountered at the McNary juvenile fish facility.  Unfortunately, this fish was a mortality removed from a raceway.    The otherwise healthy appearing specimen measured 110.2 millimeters in length.

Most research studies are at or near their conclusion.

GBT examinations concluded August 28.

Project:  Ice Harbor

# Biologist:  Mark Plummer

Dates:  August 22 - 28, 2008

## Turbine Operation

Turbine units 2, 3, 4, and 5 were available for operation all through the reporting period. Turbine unit 1 went out of service August 25 at 0800 hours for main unit cooling water strainer replacement. Turbine unit 6 remained out of service for annual maintenance.

## Adult Fish Passage Facilities

### Fish facility personnel inspected the adult fishways August 25, 26, and 28.

### Fish Ladders:  All north and south fish ladder inspection areas (picketed leads, head differentials, fishway exits, and depth over weirs) were within criteria.

### Fishway Entrances and Collection Channel:  The south shore entrance (SFE) was on sill with a depth of 7.0 feet, on sill with a depth of 7.7 feet, and on sill with a depth of 7.2 feet.  These and subsequent measurements are reported in the respective order of the inspection dates mentioned above.  The north powerhouse entrance (NFE) was on sill with a depth of 7.0 feet, on sill with a depth of 7.7 feet, and on sill with a depth of 7.2 feet.  The north shore entrance (NSE) was on sill with a depth of 6.7 feet, on sill with a depth of 6.5 feet, and on sill with a depth of 6.4 feet.   Fishway entrance criterion is 8 feet depth, greater than 8 feet depth, or on sill.  Channel/tailwater differentials were all within criteria.  Channel/tailwater differential criteria are 1 – 2 feet.

### Auxiliary Water Supply System:  2 of the 3 north fish pumps and all eight south shore fish pumps were operated without problems.  North shore fish pump 2 remains out of service.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  No problems to report.  Fish ladder exits are clear of debris and the bubblers are operating satisfactorily.

STSs/VBSs:  STSs were put placed in cycle run mode operation August 14 as average fork length of sub-yearling Chinook was greater than 120 mm at Lower Monumental Dam.  No problems were found during the August STS inspections.  September STS inspections are scheduled for September 4.

Orifices, Collection Channel, Dewatering Structure, and Bypass Pipe:   20 orifices are open.  Both orifices are closed in turbine unit 1C slot in support of gatewell bulkhead placement.  Bulkhead placement will allow project personnel to unwater the gatewell and replace cooling water strainers.  No other problems to report.

Juvenile Bypass Facility:  The bypass is in operation with 20 orifices open providing 300 cfs into the juvenile collection channel.  270 cfs is dewatered and directed into the south adult fish pump chamber for additional adult fish attraction water.

Removable Spillway Weir:  The RSW is in operation.  Sensing equipment was installed July 29 in an attempt to locate the cause of RSW vibrations.

Fish Sampling:  Juvenile fish sampling concluded July 31.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Ice Harbor Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 39.0 | 26.1 | 29 | 16 | 69 | 69 | 6.8 | 6.8 |

*Unit 1 scrollcase temperature.

Other

Turbine unit 1, slot C was dipped for fish prior to bulkhead placement on August 22 for cooling water strainer replacement.  No fish were found.

# Project:  Lower Monumental Dam

# Biologists:  Bill Spurgeon and Ken Fone

Dates:  August 22 - 28, 2008

## Turbine Operation

All turbine units are being operated in hard constraint of the 1% operation criteria as of April 1.  Turbine unit #5 was returned to service on August 27.

Adult Fish Passage Facility

The adult fishway was inspected by Corps or State biologists on August 23, 24, and 27.

Fish Ladders:

Fishway exit head differentials and depths over the weirs were within criteria (< 0.5’ and 1.0’-1.3’, respectively).  Picketed lead head differentials were in criteria (< 0.4’ and < 0.3’ for north and south shore fishways, respectively) on all inspections.

### Fishway Entrances and Collection Channel:

NSE1 and NSE2 weir gates were in depth criteria (criteria: > 8’ or on sill) through this period.  North shore channel/tailwater head was in criteria (1’-2’).

SPE1 and SPE2 weir gates were in sill criteria (criteria: > 8’, on sill, or closed during 2-pump operation) throughout this period with depths from 5.1 to 5.3 feet.    South powerhouse channel/tailwater head was within criteria (1’-2’).

SSE1 weir gate was in sill criteria (criteria: > 8’ or on sill) through this period with depths from 5.3 to 5.1 feet.  SSE2 was in criteria (6’ above sill).  South shore channel/tailwater head was in criteria (1’-2’) feet.

### Auxiliary Water Supply System:

Auxiliary water supply pumps 1 and 2 are operating.  Pump 3 is out of service to in support of diffuser assembly and bearing housing work.  Pump 3 is “bulkheaded off” to minimize water loss during two-pump operation.  Two-pump operation is expected to continue through the remainder of this operating year.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  There was an average of 0.1 square yards of forebay debris observed during this period.  There was 0 - 3% debris coverage in the gatewells.   No oil was observed.

STSs/VBSs:  STS operation was changed to cycle mode on August 2 when an average fork length of greater than 120 mm was seen in sub-yearling chinook.   August STS and VBS inspection occurred on the 4th, 5th and 6th.  No problems discovered.

Orifices, Collection Channel, Dewatering Structure, Flume:  The bypass is operating with 19 orifices open.

Collection Facility:  No problems.

Transport Summary:   Alternate day midi-tank truck transport began on August 17.  A 2.4 milligram per liter salt solution (6 lbs per 300 gallon tanker load) was used on all trips to treat and/or ease suspected outbreaks of Flexibacter columnaris

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Monumental Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature (oF)* |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 35.7 | 25.1 | 17.5 | 13.2 | 69 | 68.5 | 5+ | 3.8 |

*Scrollcase temperatures.

#### Other

Zebra mussel monitoring stations were checked on August 7 and no mussels were found.

Project:  Little Goose

# Biologists:  Eric Volkman and George Melanson

Dates:  August 22 - 28, 2008

## Turbine Operation

Turbine units 2 through 4 were available for service throughout this report period.  Turbine unit 1 was removed from service on August 19 at 0730 hours for annual maintenance and remains unavailable.  Turbine unit 5 remains out of service due to generator casing welds.  Turbine unit 6 remains out of service due to electrical grounding (water in the stator). GDACS reports indicated turbine units when operated were within the 1% criteria.

## Adult Fish Passage Facility

COE and ODFW fisheries biologists performed measured inspections of the adult fishway August 22, 26, and 28.

Fish Ladder:  All criteria were met.

Fishway Entrances and Collection Channel:  Fishway components were in criteria during the report period. Channel to tailwater head differentials during the report periods ranged between 1.20 and 1.60 feet (criteria 1.0 – 2.0 ft.).  SSE weir depths ranged between 8.10 and 8.24 feet (criteria 8.0 ft. minimum).  NPE weirs rested on sill between 5.08 and 5.34 (criteria 7.0 ft minimum or on sill), NSE weir depths ranged between 6.04 and 6.18 feet (criteria 6.0 ft. minimum). Collection channel water velocities (criteria 1.5 – 4.0 fps) ranged between 2.6 and 2.9 fps at the SSE and between 2.1 and 2.5 fps near the NSE.  Subsurface water velocities near the SSE (flowmeter) ranged between 1.8 and 2.1 fps.

Auxiliary Water Supply System:  All three fish pumps operated within criteria during the report period.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  Forebey and gatewell debris was minimal this report period.

Orifices, Collection Channel, Dewatering Structure, Flume:  The collection channel operated with 22 open orifices.

Extended Length Submersible Bar Screens/Vertical Bar Screens (ESBS/VBS):  All ESBS/VBS operated within criteria during the report period.  Drawdown measurements were conducted on turbine unit 2 trash racks and ESBS screen on August 25.  All measurements were within criteria.

Transportation Facility:  The facility continued collection for transport operations.  Daily fish smolt collection ranged between 86 and 1,037 with a total of 2,875 smolts collected for the week. Descaling remained low and ranged from 0% to 0.6%.  Observed mortality ranged between 0.2% and 2.3%.

Transport Summary:  Alternate day truck transport operations continued this report period with the 300-gallon midi tank used on August 23, 25 and 27.  Truck loading and transport to the new release facilities at the Bonneville Smolt Monitoring Facility were performed without problems.

River Conditions

Beginning August 26 at 1300 hours and continuing through August 31 at 2359 hours, spill was changed to approximately 11 KCFS using the Bulk, Alternate and uniform spill patterns provided by RCC at the request of TMT.

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Little Goose Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low | High | Low | High | Low | High | Low | High |
| 25.9 | 36.5 | 7.7 | 11.1 | 66.8 | 69.0 | 4.7 | 5.7 |

*Ladder temperature.

Other

The University of Idaho and USGS continued with adult fish passage research involving tracking of adult salmon and the monitoring of hydrologic flow patterns in the tailrace.

Piscivorous bird activity continued to be minimal.  On August 26, 9 cormorants and one gull were observed in the forebay area during daily juvenile fish passage facility inspections.

The Zebra mussel substrate monitor was inspected on August 26 – no mussels were observed.

Project:  Lower Granite

# Biologists:  Mike Halter and Kent Blevins

Dates:  August 22 - 28, 2008

## Turbine Operation

Lower Granite had turbine units 1, 4, and 5 available for power generation at the beginning of the report week.  Turbine unit 2 remained out of service for a rewind of the generator and repairs to the blade and liner.  It is projected to return to service September 30, 2008.  Turbine unit 6 is projected to return to service on September 2, 2008 following annual maintenance and a C02 (fire protection system) upgrade.  Turbine unit 3 is projected to return to service on December 31, 2008 following a 6-yr overhaul.  All operating turbine units were run within the established guidelines.

## Adult Fish Passage Facility

On August 22, 25, 26, and 28 the Lower Granite fisheries biologists performed measured inspections of the adult fishway system.  The August 26 inspection was conducted with the ODFW smolt monitoring biologist stationed at Little Goose Dam.

Fish Ladder:  All criteria were met.

Fishway Entrances and Collection Channel:  Head differential readings remained within criteria at all adult fishway entrances during this week’s inspections.  Weir depths at the south shore fishway entrances were on sill all week due to tailrace elevations at or below 633.0 feet.  Weir depths at the south shore entrances ranged from 7.3 feet to 7.8 feet (criteria ≥ 8.0 feet).  The north powerhouse fishway entrance weirs were on sill all week due to tailwater elevations well below 636.0 feet (at which level these gates bottom out).  Weir depths at the north powerhouse entrances ranged from 4.7 to 5.2 feet.   North shore fishway entrance weir gate depths were out of criteria all week and varied between 5.5 and 5.7 feet (criteria ≥7.0 feet).  Weir depth readings at the north shore entrances are being sacrificed in order to maintain the requisite 1.0 foot of head differential.  Weekly velocity readings in the adult fishway transition pool ranged from 0.93 to 1.12 feet per second and averaged 0.99 feet per second.

### Auxiliary Water Supply System:   Fish pumps 1 and 3 were operated during the week.  Fish pump 1 temporarily lost power during a power switching operation on August 27.  It was off line from 0845 to 0911 hours.  An over-voltage spike during station service switching caused the fish pumps voltage regulator to operate beyond normal range limits.

#### Juvenile Fish Passage Facility

The sample rate remained at the 100% level during the report week.

Forebay Debris/Gatewell Debris/Oil:   Debris levels were relatively low in the forebay and gatewells during the week.  There was no evidence of oil in any gatewell.

ESBSs/VBSs:   Nothing to report.  The next video inspections are scheduled for October.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:   Collection gallery orifices are being backflushed every 3 hours around the clock to prevent blockages.  It was not necessary to dewater the separator inclined screen to remove debris this week.

Transportation Facility:   The transport facility operated without any problems.  Debris levels remained low throughout the week.

Transport Summary:  Trucking operations to the release pipe below Bonneville continued during the week.  Due to relatively low fish numbers (and poundage), Lower Granite was able to truck fish using the pickup midi-tanker all week.

Removable Spillway Weir:   Nothing to report on the RSW.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Granite Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 38.10 | 31.20 | 18.6 | 18.2 | 65.2 | 66.6 | 5.0 | 4.0 |

*Scrollcase temperature.

Other

Due to lower than usual water temperatures for this time of year, the adult fish trap was returned to service during the report week.  The attraction water was turned on August 23 and the trap was put into operational mode on August 24.  The initial sample rate was set at 20%.  Scale samples are to be taken off unmarked Chinook and also off both hatchery and wild steelhead.  Tissue samples are to be taken off wild steelhead.