U.S. ARMY CORPS OF ENGINEERS

WALLA WALLA DISTRICT

#### FISH FACILITIES WEEKLY REPORT

#2-2009

# Project:  McNary

# Biologists:  Brad Eby and Bobby Johnson

Dates:  March 6 – 12, 2009

## Turbine Operation

McNary had 12 to 14 turbine units available for power generation this week.  On March 9, turbine units 1 and 2 were removed from service in support of transmission line 1 work which should be completed early next week.  All turbine units were operated within soft 1% constraints this week.  Hard 1% constraint criteria operations will begin April 1.

## Adult Fish Passage Facilities

On March 6, 10 and 12, McNary fisheries biologists performed measured inspections of the adult fishways.  Both ladders will resume visual adult fish counts on April 1.

### Fish Ladders:  During measured inspections, both ladders met all Fish Passage Plan criteria.  The Oregon ladder exit remains in manual operation as are the traveling screens in support of electrical system upgrades.  Operators are monitoring the exit weirs regularly and making adjustments as required.  Traveling screens are being cleaned every 4 hours.  Northeast winds moved woody debris into the vicinity of the Oregon ladder exit this week.  This debris is being closely monitored.

Fishway Entrances and Collection Channel:  The Washington ladder met all inspection points criteria.  Oregon ladder entrance weir NFEW2 measured 8.8 feet on March 6 (below the 9.0’depth criteria).  NFEW1 was raised March 5 as water was flowing over the top of the weir.  This entrance was closed prior to this date.  The remaining Oregon Ladder entrances (SFEW1 & SFEW2) were in criteria all week.  The north pool differential measured 0.9’ on March 10 and 12.  The south pool differential measured 0.5 feet during one fish pump operation, but otherwise met criteria this week.  Collection channel velocities average 1.9 feet per second during this report period.

Auxiliary Water Supply System:  Fish pump 3 remains unwatered and out of service for the oil leak reported during the last season.  Fish pumps 1 and 2 operated all week with blade angles of 30 degrees except on March 9 and 10.  On March 9, fish pumps were switched to a different electrical bus in support of transmission line 1 work.  After the switch, blade angles on both operating pumps were adjusted along with the pump 2 exciter.  On March 10, pump 2 was taken out of service so a new breaker could be installed.  Both outages lasted less than one hour.  The juvenile facility is not supplying the usual 450 cfs to the north powerhouse pool since the juvenile facility is currently out of service.  The Wasco County PUD turbine unit had no interruptions in service this week.

#### Juvenile Fish Passage Facility

The juvenile facility remains shut down for winter maintenance, which is nearing completion.

Forebay Debris/Gatewell Debris/Oil:  Forebay debris was light to moderate depending on wind direction.  Preseason trash rack cleaning is scheduled to take place next week.    No problems were encountered this week.

ESBSs/VBSs:  ESBS maintenance is nearly completed and screens remain in their raised positions.  Although ESBS maintenance has been slowed by severe winter weather earlier this year, installations will occur between April 6 and April 16 as planned.  ESBS deployments will begin one to two weeks later than in past years to reduce impacts on juvenile lamprey passage.  VBS monitoring will resume once ESBS installations begin.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:  Winter maintenance is near completion.  Bypass operations will begin concurrently with fish screen installations.

Transportation Facility:  Winter maintenance is near completion.  A 10 minute facility power outage took place March 9 in support of the transmission line outage mentioned above.  Facility operations were minimally impacted.

Transport Summary:  Drivers, engineers and biologists met at the Bonneville Truck Pad on March 10, to discuss potential improvements.  Recommended changes will be made prior to the start of trucking operations.

River Conditions

River conditions during the week are outlined in Table 1 as provide by control room data, which runs from 0000 to 0000 each day.

Table 1.  River conditions at McNary Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 110.4 | 100.8 | 0.0 | 0.0 | 40 | 39 | 6.0 | 6.0 |

*Unit 1’s scroll case temperature.

Other

Bird counts will resume later in the month when technicians are on duty.  An occasional gull, grebe, cormorant, or merganser was observed during ladder inspections.  A few birds of each species now appear to over winter near the project.  USGS researchers have nearly completed antenna and wiring installations for the upcoming season.

Project:  Ice Harbor

# Biologist:  Mark Plummer and Stephen Jeffers

Dates:  March 6 – 12, 2009

## Turbine Operation

All turbine units (1 - 6) were available for operation during this reporting period.  Turbine unit 6 was out of service from March 9 at 0800 hours to March 12 at 1735 hours in support of RSW-related dives.

## Adult Fish Passage Facilities

### Fish facility personnel inspected the adult fishways March 10, 11, and 12.

### Fish Ladders:  All north and south fish ladder inspection areas (picketed leads, head differentials, fishway exits, and depth over weirs) were within criteria.   North and south fish ladder picketed leads are currently raised.

### Fishway Entrances and Collection Channel (results in inspection date order):  The south shore entrance (SFE) was off sill with a depth of 8.5 feet, off sill with a depth of 8.6 feet, and off sill with a depth of 8.5 feet.  The north powerhouse entrance (NFE) was off sill with a depth of 8.9 feet, off sill with a depth of 10.1 feet, and off sill with a depth of 9.5 feet.  The north shore entrance (NSE) was off sill with a depth of 7.1 feet, off sill with a depth of 7.3 feet, and off sill with a depth of 8.1 feet.  The north shore fish entrance sill elevation was not properly set or calibrated within fishway software.  Once the correct sill elevation programmed in the PLC (programmable logic circuit) softway, the weir lowered into criteria.  Fishway entrance criterion is 8 feet depth, greater than 8 feet depth, or on sill.  All channel/tailwater differentials were in criteria this week.  Channel/tailwater differential criteria are 1 – 2 feet.

### Auxiliary Water Supply System:  Two of the 3 north shore fish pumps were operated without any problems.  North shore fish pump 3 is out of service, and has been taken off site for repairs.  The expected return and reinstallation date is tentatively March 23.  All (8) south fish pumps were available for service.  Currently, 6 pumps are in operation.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  No problems to report.  Some debris is accumulating in the forebay.  Fish ladder exits are clear of debris and the bubblers are operating.  Turbine unit trash rack raking is scheduled March 23 to 31.

STSs/VBSs:  STSs are currently raised for annual maintenance and are tentatively scheduled to be installed March 23 – 31.

Orifices, Collection Channel, Dewatering Structure, and Bypass Pipe:   The juvenile fish bypass is not in operation and all orifices are closed. Start up is tentatively scheduled for March 16.

Juvenile Bypass Facility:  The bypass is scheduled to be put into operation March 16.

Fish Sampling:  The first sample is scheduled for April 3.

Removable Spillway Weir: The RSW is currently in the stowed (away from the dam) position as repairs to the seals, fish tracking antenna installation, and a small completion contract are in progress.  Expected completion date is March 16.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Ice Harbor Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 41 | 31.8 | 0.0 | 0.0 | 40 | 38 | 7.2 | 7.0 |

*Unit 1 scrollcase temperature.

Other

Navigation lock outage began March 7 and completion is expected by March 21.

Project:  Lower Monumental Dam

# Biologists:  Bill Spurgeon and Ken Fone

Dates:  March 6 – 12, 2009

## Turbine Operation

Turbine units are being operated in the soft constraint of the 1% operation criteria.  Turbine units 5 and 6 were out of service intermittently from March 9 to March 12 to accommodate RSW testing and research equipment installation.

Adult Fish Passage Facility

The adult fishway was inspected by Corps biologists on March 10 and 12.

Fish Ladders: Fishway exit head differentials and depths over the weirs were within criteria (< 0.5’ and 1.0’-1.3’, respectively) on all inspections.  Picketed lead head differentials were in criteria (< 0.4’ and < 0.3’ for north and south shore fishways, respectively) on all inspections.

### Fishway Entrances and Collection Channel: NSE1 and NSE2 weir gates were in depth criteria (criteria: > 8’ or on sill) on all inspections.  North shore channel/tailwater head was in criteria (1’-2’) on all inspections.

SPE 1 and SPE 2 weir gate was in sill criteria (criteria: > 8’ or on sill) with a depth of 7.5’, 7.6’ and 7.7 feet.  South powerhouse channel/tailwater head was in criteria (1’-2’) this week.

SSE1 weir gate was in depth criteria (criteria: > 8’ or on sill) through this period.  SSE2 was in criteria (6’ above sill) on all inspections.  South shore channel/tailwater head was in criteria (1’-2’) this week.

### Auxiliary Water Supply System: AWS pump 3 is out of service to replace the bearing housing and repair the diffuser assembly.  Pump 3 is “bulkheaded off” to minimize water loss during two-pump operation.   Pumps 1 and 2 remain in service.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil: There was an average of 185 square yards of forebay debris observed during this period.  Debris will be removed from the gatewells and trash racks during the week of March 16.

STSs/VBSs: STSs will be installed during the week of March 23. The STS rebuild continues.

Orifices, Collection Channel, Dewatering Structure, Flume:  The bypass will be watered up during the week of March 23.

Collection Facility:  Winter maintenance is being finished up.  The facility will be watered up for testing the week of March 23.

Transport Summary:  Not collecting for transport.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Monumental Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature (oF)* |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 40.0 | 31.5 | 0 | 0 | 40.0 | 39.5 | 2.7 | 2.5 |

*Scrollcase temperatures.

#### Other

Spillgates 3-8 and Turbine units 5 and 6 were intermittently removed from service from March 9 through 12 to accommodate dive work in support of RSW testing and research equipment installation.

There were no zebra mussels observed at the monitoring stations this month.

Project:  Little Goose

# Biologists:  Eric Volkman and George Melanson

Dates:  March 6 – 12, 2009

## Turbine Operation

Turbine units 1 through 5 were available for service throughout the week.  Turbine unit 6 was returned to service on March 10 at 1250 hours following TSW installation.  Turbine unit 6 was forced out of service on March 12 at 1215 hours due to grounding errors.  All turbine units are being operated in soft constraint of the 1% efficiency criteria.

## Adult Fish Passage Facility

USACE fisheries biologists performed measured inspections of the adult fishway March 10, 11 and 12.

Fish Ladder:  All criteria were met.

Fishway Entrances and Collection Channel:  All fishway components were in criteria.  During inspections, channel to tailwater head differentials ranged between 1.14 and 1.62 (criteria 1.0 ft.).  SSE weir depths ranged between 8.28 and 8.38 feet (criteria ≥8.0 ft), NPE weir depths ranged between 7.24 and 7.32 feet (criteria ≥7.0 ft.) and NSE weir depths ranged between 6.02 and 6.14 feet (criteria ≥ 6.0 ft.).  Collection channel water velocities (criteria 1.5 fps) ranged between 2.2 and 2.7 fps near the SSE and 1.6 fps near the NSE.   Subsurface water velocities near the SSE (flowmeter) ranged between 1.8 and 2.1 fps.

Auxiliary Water Supply System:  All three fish pumps operated within criteria.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  Up to 3,000 square feet of floating debris was observed inside the trash-shear boom in front of turbine units 1, 2 and 3 intakes.

ESBSs/VBSs:  All ESBSs remained raised and in their winter storage positions.  Maintenance is ongoing.

Orifices, Collection Channel, Dewatering Structure, Flume:  The juvenile collection system is out of service for winter maintenance.

Transportation Facility:  The facility is out of service for winter maintenance.

Transport Summary:  Transport operations are expected to begin after April 20.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Little Goose Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 37.9 | 30.6 | 0.5 | 0 | 40.0 | 39.4 | 2.8 | 2.5 |

*Ladder temperature.

Other

Advanced American contractors have completed installation of spillway deflectors in spillway 1 and 8 and the TSW in spillway 1.

TSW flow test was conducted on March 11 from 0800 to 0931.  Additional spill was conducted for 1 hour in the afternoon to test research equipment.

HARCON contractors have installed the full flow flume PIT–tag detector during the winter maintenance period.  HARCON is continuing work on upgrades to the primary dewaterer and flume outfall relocation.

Normandeau Inc., - A direct injury study is scheduled to begin March 19.

Project:  Lower Granite

# Biologists: Mike Halter and Kent Blevins

Dates:  March 6 – 12, 2009

## Turbine Operation

Lower Granite had turbine units 1 – 6 available for power generation at the beginning of the report week.

## Adult Fish Passage Facility

On March 10, 11 and 12 the Lower Granite fisheries biologists performed measured inspections of the adult fishway system.

Fish Ladder:  All criteria were met.

Fishway Entrances and Collection Channel:  Head differential readings were within criteria at all adult fishway entrances during the weekly inspections.  Weir depths at the south shore fishway entrances were within criteria on the March 11 and 12 inspections but slightly out of criteria on the March 10 when SSE-1 had a reading of 7.9 feet (criterion ≥ 8.0 feet).  Weir depths at the north powerhouse fishway entrances were either in criteria or slightly out of criteria on all inspections with readings ranging between 7.9 and 8.0 feet (criterion ≥ 8.0 feet).  Weir depths at the north shore fishway entrances were out of criteria all week with readings ranging from 5.1 to 5.3 feet (criterion ≥ 7.0 feet).  Weir depth readings at the north shore entrances are being sacrificed in order to maintain the requisite 1.0 foot of head differential.  Weekly velocity readings in the adult fishway collection channel transition pool area ranged from 0.88 to 0.99 feet per second and averaged 0.90 feet per second.

### Auxiliary Water Supply System:  Fish pumps 1 and 3 were run during the week without any problems.

#### Juvenile Fish Passage Facility

The juvenile collection/bypass system has been shut down for the winter outage since December 13, 2008.  Water up of the collection gallery and initiation of primary bypass at the separator is tentatively scheduled for March 20.  Sampling activities are scheduled to begin on March 25.

Forebay Debris/Gatewell Debris/Oil:  Forebay debris was minimal and none was removed this week.  Trash was raked from all units in late February.

ESBSs/VBSs:  The ESBSs were dogged-off for maintenance work during the week.  Deployment of ESBSs is tentatively scheduled to begin during the third week of March.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:  Nothing to report at this time.

Transportation Facility:  General transport operations will not begin until sometime between April 20 and May 1.  Research barging operations will begin sometime in early April.  Juvenile fish sampling (only) activities are tentatively scheduled to begin on March 25.

Transport Summary:  Nothing to report.

Removable Spillway Weir:   Nothing to report on the RSW.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Granite Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 45.8 | 32.5 | 0.0 | 0.0 | 40.0 | 39.5 | 3.5 | 2.7 |

*Scrollcase temperature.

Other

Video counts in the adult fish ladder counting room began on March 1.  The recording hours are from 0600 to 1600 hours.

The adult fish trap operations resumed on March 3 at 1121 hours with a sample rate of 10%.