U.S. ARMY CORPS OF ENGINEERS

WALLA WALLA DISTRICT

#### FISH FACILITIES WEEKLY REPORT

#6-2009

# Project:  McNary

# Biologists:  Brad Eby and Bobby Johnson

Dates:  April 3 – 9, 2009

## Turbine Operation

McNary had 14 turbine units available for power generation this week.  On April 5, turbine unit 5 was taken out of service for 4 hours so an exciter relay could be replaced.  From April 7 through 9, turbine units 1 through 6 were individually taken out of service for a cumulative total of 28.8 hours in support of ESBS installation and semi-annual maintenance.  During this period, water was also removed from the blade hub in turbine unit 6 (also known as “hub taping”).  All turbine units operated within the hard 1% criteria which began April 1.

## Adult Fish Passage Facilities

McNary fishery biologists performed measured inspections of the adult fishways on April 4, 6 and 9.  Visual adult fish counting is in progress.

### Fish Ladders:  Both fish ladders met all Fish Passage Plan criteria.  The Oregon weirs and traveling screens remain in manual mode as electrical system upgrades continued.  Operators are monitoring the exit weirs regularly and making exit weir adjustments as needed and cleaning traveling screens every 4 hours.  On April 7, the Oregon ladder tilting weirs were calibrated and brakes associated with weir 339 brakes were found in need of replacement.  The Washington ladder weir 337 triggered an exit alarm, which was reset without incident.

Fishway Entrances and Collection Channel:  The Washington ladder met all inspection point criteria.  All Oregon ladder criteria were met except for NFEW2 (8.9’ on April 4).  Collection channel velocity averaged 1.3 feet per second.

Auxiliary Water Supply System:  Fish pump 3 remains out of service due to the oil leak discovered during the last season.  This pump is currently un-watered and repair work is progressing.  Pumps 1 and 2 ran all week with no interruptions in service with blade angles remaining at 30 degrees.  The juvenile facility continued to supply the usual 450 cfs to the north powerhouse pool.  The Wasco County PUD unit had no interruptions in service this week.

#### Juvenile Fish Passage Facility

On April 7, the juvenile fish facility was placed in secondary bypass for 13 minutes even though formal sampling had not yet begun.  This enabled the testing of the GBT program water supply pump (following repairs) and technician training. Sample and PIT tag gates were inoperative during this operation.  One unclipped pre-spawn steelhead adult was returned to the river from the separator during this time.  Spring primary bypass continued until 0700 on April 8 when secondary bypass began.  This was the first day secondary bypass could occur following ESBS installation.  The spring bypass season will now consist of alternating days of primary and secondary bypass with the switch occurring every morning at 0700 hours.  The first full day of secondary bypass operations (which ended April 9), passed 410 smolts through the facility.

Forebay Debris/Gatewell Debris/Oil:  Forebay debris was light to moderate.  Trash rack differentials continue to be monitored and no cleaning occurred.  No problems were observed in the gatewell slots.

ESBSs/VBSs:  ESBS’s installation began on April 7 instead of April 6 as orifginally planned as spill bay hoist issues needed to be resolved.  From April 7 to 9, fish screens were installed at units 1 through 6 without incident.  On April 8, a spare fish screen was moved from storage to the forebay deck.  ESBS installations should be completed by April 16.  Fish screen camera inspections will begin in late April.  VBS monitoring resumed April 7 and no problems were observed this week.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:  The number of open orifices remained at 42 orifices throughout the week as due to stable forebay elevations.  All channel equipment remained in automatic operation.  No problems were noted except for the transition screen cleaning mechanism which was reset once.  The transition cleaning mechanism continues to be turned off at night to prevent excessive wear and tear.  During primary bypass, the fisheries staff continued to monitor the channel regularly around the clock.  During secondary bypass operations the technician never leaves the separator unattended and the collection channel continues to be monitored by alarms when additional staff is unavailable (i.e.: night shifts).

Transportation Facility: Full flow PIT tag detection occurred during primary bypass operations and throughout the facility during secondary bypass operations.  Smolt monitoring only occurred during secondary bypass operations.  Following tests, the primary (A and B) PIT tag slide gates were turned off and closed as the return to river lines provide a better bypass route for PIT tagged fish.  Sample gates were activated April 8 and will continue to be functional only during secondary bypass days.  All systems including the sample gates functioned well this week except for difficulties noted and resolved on April 8.  These included a small blockage in the A-side return to the line (ice blocks removed the obstruction) and 2 malfunctioning fish counters (replaced).  Subsequent tests revealed that the replacement counters were operating properly.  No other problems were observed at the facility.

Transport Summary:  Fish transport did not take place this week.

River Conditions

River conditions during the week are outlined in Table 1 with clarity and temperature provided by control room.  Flows and spill were provided by the smolt monitoring staff whose data day runs from 0700 to 0700 hours.  The smolt monitoring staff will start providing temperature data next week.  Also, the operators received gas cap and spill criteria.  On April 10 at 0001 hours, spill for juvenile fish passage will begin with 40 percent of flow being spilled.

Table 1.  River conditions at McNary Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 147.1 | 132.6 | 0.0 | 0.0 | 45 | 43 | 2.3 | 2.1 |

*Unit 1’s scroll case temperature.

Other

Predatory bird counts continue with the tailwater and forebay areas being checked twice and once a day, respectively.  The tailwater area had a high count of 18 gulls with an occasional pelican or cormorant also being observed.  Occasionally, a gull was noted by the bypass outfalls.  The water cannon continued to operate well and bird hazing personnel are on duty.  In forebay areas, grebes, cormorants, gulls and ospreys were occasionally noted.

No zebra or quagga mussels were noted at the monitoring stations this week.

Spillbay and TSW hoists operations and maintenance issues were resolved this week.  Both TSWs and all spillbays are available for service.

Project:  Ice Harbor

# Biologist:  Mark Plummer

# Technician: Stephen Jeffers

Dates:  April 3 – 9, 2009

## Turbine Operation

All turbine units (1 - 6) were available for operation this reporting period.  Turbine units 1 and 2 were out of normal operating priority criteria due to an unexpected outage of transmission line 1 on April 6 from 0955 hours to 1004 hours.

## Adult Fish Passage Facilities

### Fish facility personnel inspected the adult fishways on April 6, 7, and 10.

### Fish Ladders:  All north and south fish ladder inspection areas (picketed leads, head differentials, fishway exits, and depth over weirs) were within criteria.   North and south fish ladder picketed leads were lowered March 30.

### Fishway Entrances and Collection Channel (inspection date order):  The south shore entrance (SFE) was off sill with a depth of 8.8 feet, off sill with a depth of 8.6 feet, and off sill with a depth of 8.1 feet.  The north powerhouse entrance (NFE) was on sill with a depth of 11.2 feet, on sill with a depth of 11.2 feet, and on sill with a depth of 12.3 feet.  The north shore entrance (NSE) was off sill with a depth of 8.5 feet, off sill with a depth of 8.5 feet, and off sill with a depth of 8.8 feet.  Fishway entrance criterion is 8 feet depth, greater than 8 feet depth, or on sill. All channel/tailwater differentials were in criteria.  Channel/tailwater differential criteria are 1 – 2 feet.

### Auxiliary Water Supply System:  2 of the 3 north shore fish pumps were operated this week.  North shore fish pump 3 returned to service April 8 at 1400 hours following the completion of major repairs.  Seven south shore fish pumps were available for operation.  Of these, 6 are currently in operation.  South fish pump 5 is out of service for gear box repairs.  North fish pumps 1 and 2 along with south fish pumps 1, 6, and 7; were taken out of service April 6 from 0552 hours to 0652 hours due to an unexpected outage of transmission line 1.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  No problems to report.  Fish ladder exits are clear of debris and the bubblers are operating.

STSs/VBSs:  STSs are in cycle mode operation.

Orifices, Collection Channel, Dewatering Structure, and Bypass Pipe: 20 orifices are open.

Juvenile Bypass Facility:  The bypass is in operation with 20 orifices open providing 300 kcfs into the juvenile collection channel.

Fish Sampling:  The first sample was performed April 7.

Removable Spillway Weir:  The RSW is in the operating position.

Fish Sampling:

April 7

| Species | Sampled | #Descaled | Morts | Avian Marks |
| --- | --- | --- | --- | --- |
| C-CH | --- | --- | --- | --- |
| UC-CH | 9 | 0 | 0 | 0 |
| C-CH-O | --- | --- | --- | --- |
| UC-CH-O | --- | --- | --- | --- |
| C-SH | --- | --- | --- | --- |
| UC-SH | 1 | 0 | 0 | 0 |
| C-COHO | --- | --- | --- | --- |
| UC-COHO | --- | --- | --- | --- |
| C-SOCK | --- | --- | --- | --- |
| UC-SOCK | --- | --- | --- | --- |
| TOTAL | 10 | 0 | 0 | 0 |

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Ice Harbor Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 88.4 | 63.6 | 59.5 | 0.0 | 44 | 43 | 2.3 | 1.9 |

*Unit 1 scrollcase temperature.

Other

Voluntary spill for fish began April 3.

Project:  Lower Monumental Dam

# Biologists:  Bill Spurgeon and Ken Fone

Dates:  April 3 – 9, 2009

## Turbine Operation

Turbines units are being operated within the 1% operation criteria.  Turbine unit 5 was out of service on April 7 from 0715 to 1055 hours to replace the thrust bearing alarm switch.

Adult Fish Passage Facility

The adult fishway was inspected by Corps or state biologists on April 3, 4, 8 and 9.

Fish Ladders: Fishway exit head differentials and depths over the weirs were within criteria (< 0.5’ and 1.0’-1.3’, respectively) on all inspections.  Picketed lead head differentials were in criteria (< 0.4’ and < 0.3’ for north and south shore fishways, respectively) on all inspections.

### Fishway Entrances and Collection Channel: NPE 1 and NPE 2 weir gates were in depth criteria (criteria: > 8’ or on sill) on all inspections.  North shore channel/tailwater head differential was in criteria (1’-2’) on all inspections.

SPE 1 and SPE 2 weir gates were in sill criteria (criteria: > 8’ or on sill) with a depths of 6.2’ to 7.6 feet.  South powerhouse channel/tailwater head differential was in criteria (1’-2’) on all inspections.

SSE1 weir gate was in depth or sill criteria (criteria: > 8’ or on sill) through this period.  Depths while on sill ranged from 6.6’ to 7.5 feet.  SSE2 was in criteria (6’ above sill) on all inspections.  South shore channel/tailwater head was in criteria (1’-2’) on all inspections.

### Auxiliary Water Supply System: AWS pump 3 remains out of service for bearing housing replacement and diffuser assembly repairs.  This work is scheduled to be completed late in 2009.  Pumps 1 and 2 were in service throughout this period.   Pump 3 is “bulkheaded off” to minimize water loss during two-pump operation.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  There was an average of 0 square yards of forebay debris observed during this period.  Debris coverage in gatewells 4B and 4C repectively reached 50% on April 4 and 55% on April 3.  Debris was removed from gatewells on April 6 as gatewell debris coverage criterial is 50% or less.  No oil was observed in gatewells this week.

STSs/VBSs:  The STS rebuild continues.  STSs were installed on March 24 and 25 and are operating in cycle mode.

Orifices, Collection Channel, Dewatering Structure, Flume:   The bypass is watered up with 19 orifices open.

Collection Facility:  Collection of fish for condition monitoring began on April 1 and avian predation researchers added their fish collection requirements on April 5.

Transport Summary:  Not collecting for transport at the present time.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Monumental Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature (oF)* |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 85.1 | 59.1 | 32.0 | 25.2 | 45.5 | 43.5 | 2.4 | 2.0 |

*Scrollcase temperatures.

#### Other

Turbine unit cooling water strainers were cleaned on April 8.  Forty-one dead juvenile lampreys were collected from the strainers.  We plan to collect this information monthly through this year.

There were no zebra mussels observed at the monitoring stations on April 3.

Project:  Little Goose

# Biologists:  Eric Volkman and George Melanson

Dates:  April 3 – 9, 2009

## Turbine Operation

Turbine units 1 through 4 were available for service throughout the week.  Turbine units 5 and 6 remained out of service through the week due to an electrical bus grounding error (both turbine units share the same electrical bus).  An additional clearance was placed on turbine unit 5 on April 1 in support of fish screen repairs. All available turbine units were operated within 1% efficiency criteria this week.

## Adult Fish Passage Facility

USACE and ODFW fisheries biologists performed measured inspections of the adult fishway on April 6 and 7.

Fish Ladder:  All criteria were met.

Fishway Entrances and Collection Channel:  All fishway components were in criteria during the report period.  During inspections, channel to tailwater head differentials ranged between 1.14 and 1.80 (criteria 1.0 -2.0 ft.).  SSE weir depths ranged between 8.20 and 8.29 feet (criteria ≥8.0 ft), NPE weirs rested on sill, depths measured between 5.58 and 5.62 feet (criteria ≥7.0 ft. or on sill) and NSE weir depths ranged between 6.10 and 6.16 feet (criteria ≥ 6.0 ft.).  Collection channel water velocities (criteria 1.5 fps) ranged between 2.5 and 3.1 fps near the SSE.  Subsurface water velocities near the SSE (flowmeter) ranged between 1.9 and 2.2 fps.

Auxiliary Water Supply System:  All three fish pumps operated within criteria.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  Up to 150 square feet of floating debris was observed inside the trash-shear boom in front of turbine units 1, 2 and 3 intakes.

ESBSs/VBSs:  ESBSs in intake slots 5B, 5C and 6A were removed for repair on April 1 and are scheduled to return to service on April 13.

Orifices, Collection Channel, Dewatering Structure, Flume:  The collection channel operated with 20-21 opened orifices.  Orifice lights 1A through 2A were repaired and returned to service on April 8.

Transportation Facility:  Collection continued with 24 hour sample and secondary by-pass.  All components are operating in criteria.

Transport Summary:  Transport operations are expected to begin after April 20.

TSW:  The TSW was placed into operation on April 3 in low crest configuration (618 ft.).

River Conditions

Spring spill operations for juvenile fish passage commenced April 3 at 0001 hours.  Spill criterion is set at 30% of total river flow (continuous) not to exceed gas saturation set at 120% in the tailrace.  Minimum operating pool (MOP) criteria took effect this report period.

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Little Goose Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 79.7 | 53.9 | 23.8 | 16.25 | 46.3 | 43.7 | 2.6 | 2.2 |

*Ladder temperature.

Other

The Zebra Mussel substrate was inspected on April 6.  No mussels were observed.

On April 1, USDA–APHIS personnel initiated piscivorous bird hazing activities.  Up to 9 gulls were observed in the vicinity of the Dam - mostly in the immediate tailrace area.

USGS continued preparations for the juvenile fish passage and survival study expected to commence April 16.

Project:  Lower Granite

# Biologist: Mike Halter

Dates:  April 3 – 9, 2009

## Turbine Operation

Lower Granite had turbine units 1 – 6 available for power generation at the beginning of the report week.

## Adult Fish Passage Facility

On April 3, 4, and 6 the Lower Granite fisheries biologists performed measured inspections of the adult fishway system.

Fish Ladder:  All criteria were met.

Fishway Entrances and Collection Channel:  Head differential readings were within criteria at all adult fishway entrances during the weekly inspections.  Weir depths at the south shore fishway entrances either met criteria or were very close to criteria during the week with depths ranging between 7.8 feet and 8.0  (criterion ≥ 8.0 feet).  Weir depths at the north powerhouse fishway entrances ranged between 6.3 and 6.7 feet during the week due to tailwater elevations below 636.0 feet.  (Whenever tailwater elevations are below 636.0 feet the north powerhouse entrances bottom out and rest on their sills.)  Weir depths at the north shore fishway entrances were out of criteria all week with readings ranging from 5.0 to 5.1 feet (criterion ≥ 7.0 feet).  Weir depth readings at the north shore entrances are being sacrificed in order to maintain the requisite 1.0 foot of head differential.  Weekly velocity readings in the adult fishway collection channel transition pool area ranged from 0.85 to 1.05 feet per second and averaged 0.93 feet per second.

### Auxiliary Water Supply System:  Fish pumps 1 and 3 were run during the week without any problems.  Bearing repair work continued on fish pump 2.  Fish pump 2 should be available for service in the near future.

#### Juvenile Fish Passage Facility

The facility sample rate was decreased from 10% to 5% at 0700 hours on April 6.  Fish collection activities for research barging operations (PIT-tagging by NMFS) began on the morning of April 6.

Forebay Debris/Gatewell Debris/Oil:  Forebay debris was minimal and none was removed this week.  Gatewells are being checked for debris and hand-dipped as circumstances warrant.

ESBSs/VBSs:  All ESBS deployments took place during the previous report period.  There were no ESBS related problems during the week.   The brush control circuits for the ESBSs in turbine units 5 and 6 continued to be manually operated every 4 hours.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:  Nothing to report at this time.

Transportation Facility:  The first research barge of the season departed Lower Granite on April 9.  Subsequent trips are planned for April 16 and April 23 – every Thursday until normal barging operations begin.

Transport Summary:  Nothing to report at this time.

Removable Spillway Weir:   Nothing to report on the RSW.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Granite Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 79.8 | 59.4 | 20.6 | 20.1 | 45.6 | 44.5 | 1.9 | 1.5 |

*Scrollcase temperature.

Other

Visual fish counting operations at the adult fish ladder counting station began on April 1.

Adult fish trap operations continued during the week with a sample rate of 10%.

Kelt sampling activities at the Lower Granite separator continued during the week.  Kelts are being collected on Sundays and Wednesdays for UI researchers to process on Mondays and Thursdays.

NMFS began juvenile fish marking operations for the transport study and the survival study during the week.  Fish were marked on April 7 and 8.  The first research barge of the season departed Lower Granite on the morning of April 9.  Fish are being released below Bonneville Dam near the Skamania Light.

No evidence of zebra mussels was found during the week.