U.S. ARMY CORPS OF ENGINEERS

WALLA WALLA DISTRICT

#### FISH FACILITIES WEEKLY REPORT

#24-2009

# Project:  McNary

# Biologists:  Brad Eby and Bobby Johnson

Dates:  August 7 - 13, 2009

## Turbine Operation

McNary had 10 to 11 turbine units available for power generation this week.  Turbine units 7 and 8 remain out of service for replacement of transformer T4.  This work should be completed in the near future.  Turbine unit 2 remains out of service until mid September for nine year overhaul.  On August 10, turbine unit 1 was removed from service for annual maintenance.  Other outages took place on August 11 (turbine units 9 and 11, ESBS camera inspections, 39 minutes total outage time) and August 13 (turbine unit 13, PPT ground work, 7 hours outage time).  The PPT or power potential transformer is part of the exciter system that initiates generator operation.  North powerhouse priority loading remains in effect to moderate juvenile collection channel and gatewell temperatures.  All available turbine units operated within the hard constraint one percent criterion this week.

## Adult Fish Passage Facilities

On August 7, 10 and 13, McNary fisheries biologists performed measured inspections of the adult fishways.  Adult fish counts, daily temperature monitoring and lamprey research continues.  On August 13, though these locations were in criteria, the Oregon exit head over weir differential and count station differential were not recorded.

Fish Ladders:  Both ladders met all Fish Passage Plan criteria during measured inspections.  Count station picketed leads in both ladders required repeated cleanings this week due to excessive milfoil accumulations.

At the Oregon ladder exit, the south traveling screen remains in automatic operation and the north traveling screen remains out of service.  The south screen operated without any problems this week and no significant changes were observed in trash rack or screen differentials.  A dive is scheduled for August 17 and 18 to examine the north screen and clean the trash racks.  The Washington exit triggered an alarm once this week and normal operation resumed after the alarm was reset.  Debris and differential monitoring efforts will continue.

Fishway Entrances and Collection Channel:  The Washington ladder met all inspection point criteria.  Other than both entrances being calibrated on August 13, no unusual operating conditions were observed this week.

Most  Oregon ladder inspection points met criteria.  Locations not meeting criteria included the north powerhouse pool differential on August 7 and 10 (respectively, differentials were 0.6 and 0.7 feet), NFEW2 and NFEW3 on August 13 (both weir depths measured 8.8 feet) and SFEW1 on August 7 and 10 (both weir depth measurements were 8.9 feet).  Weir depth set points at the north and south entrances were slightly changed to 9.15 feet on August 13.  This will help keep the weirs in criteria.  Collection channel velocities averaged 1.6 feet per second this week.

Auxiliary Water Supply System:  Fish pump 3 returned to service this week following removal of the associated intake bulkheads on August 12 and the discharge bulkheads on August 13 at 1012 hours.  Fish pumps 1 and 2 ran all week with blade angles of 30 degrees except on August 13 for three hours when the angles were set to zero in support of fish pump 3 reactivation.  The juvenile facility continues to supply the usual 450 cfs to the north powerhouse pool and the Wasco County PUD unit had no interruptions in service this week.

#### Juvenile Fish Passage Facility

Fish collection for daily barge transport continued.  Barges loaded on odd numbered days began their trips at Lower Granite Dam and barges loaded on even numbered days began their trips at McNary Dam.  For the report week, 17,600 smolts were collected for transport.  Daily facility mortality rates for the same period ranged from 0.3 to 1.7 percent.  There were also 10,390 juvenile shad collected.  Fish collection for daily barge transport is expected to continue until August 15, with truck transport expected to commence on August 16.

Forebay Debris/Gatewell Debris/Oil:  Forebay debris remains very light though some woody debris and Eurasian milfoil remain near the Oregon ladder exit.  Trash rack differentials remained low and no trash racks were cleaned this week.  Gatewell slots 7B, 7C, 8A and 8B remain covered to prevent contamination should a spill take place during the course of T4 transformer replacement.

ESBSs/VBSs:  ESBSs are currently deployed in all turbine units.  Screens in slots 11C and 13A remain in transducer bypass mode.  The screen in slot 9C triggered an alarm once the week and resumed normal operation after the alarm was reset.  ESBS video camera inspections were conducted in turbine units 9 and 11 this week and one abnormality was observed.  The ESBS cleaning brush in slot 11C was stopping 3 feet short of the top of the screen.  The required adjustment will be made in the near future.  VBS monitoring revealed no screens out of criteria and no screens were cleaned this week.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:  For the week, there were 42 orifices opened.  No problems were encountered and all automated systems operated well.

Transportation Facility:  Fish collection for barge transport continued.  The primary (A and B) PIT tag diversion gates are in service and the secondary (C and D) PIT/bypass gates remain closed.  Sample gates operated satisfactorily this week. Maintenance this report period included water balloon tests of the return to the river lines (no ruptured balloons means no obstructions or debris) and the servicing of all raceway release valves.

Transport Summary:  Barge departures took place daily and all barges were loaded without incident.  The towboat and barge had some difficulties maneuvering in and traversing the tailrace on August 11 as turbine unit 1 was not available to counteract eddies.  Otherwise, there were no problems to report.  Preparations for truck transport continue.

River Conditions

River conditions during the week are outlined in Table 1 with data provided by the smolt monitoring staff whose data day runs from 0700 to 0700 hours.  Water clarity data continues to be provided by the control room.  The smolt monitoring staff continues gathering water temperature data throughout the project except for the covered gatewell slots (mentioned above) that no longer have temperature probes installed.  Temperature information will be distributed in a separate weekly report.

Spill for juvenile fish passage continues at 50 percent of total flow.  As river flow decreases, the number of spill pattern changes for safe navigation to and from the locks has increased.  Spillway gates continue to be opened and closed for towboat and fish barge arrivals and departures.

Table 1.  River conditions at McNary Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature (oF) |  | Water Clarity* (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 139.8 | 108.0 | 68.5 | 51.9 | 71.7 | 69.1 | 6.0 | 6.0 |

*From control room data.

Other

Inline Cooling Water Strainers:  Cooling water strainers were not inspected this week.

Invasive Species:  On August 10, the zebra mussel stations were examined and no mussels were found.  The next inspection is scheduled for approximately September 10.

Predatory bird counts continued with the tailwater and forebay areas being checked twice and once a day, respectively.  The tailwater area had high counts of 77 terns, 55 gulls and 19 cormorants with an occasional merganser or pelican being noted.  Most cormorants were seen roosting in the afternoon.  No birds were seen at the bypass outfalls.  The water cannon continued to operate well.  Only an occasional tern, gull, cormorant, grebe, osprey or pelican were seen in the forebay.  No grebes were noticed in the gatewells or in the juvenile fish collection gallery this week.

Only the juvenile and adult lamprey studies are in progress.  USGS researchers have begun removing their equipment.

Project:  Ice Harbor

# Biologist:  Mark Plummer

# Technician: Stephen Jeffers

Dates:  August 7 - 13, 2009

## Turbine Operation

Turbine units 1 – 3 were available for operation throughout the entire reporting period.  Turbine units 5 and 6 went out of service August 3 for annual maintenance.  Turbine unit 4 was taken out of service August 12 to secure a loose electrical connection in the transformer canyon.

## Adult Fish Passage Facilities

### Fish facility personnel inspected the adult fishways August 10, 11, and 13.

### Fish Ladders:  All north and south fish ladder inspection areas (picketed leads, head differentials, fishway exits, and depth over weirs) were within criteria.  The south picketed leads are requiring frequent cleaning due to plugging with aquatic vegetation.

### Fishway Entrances and Collection Channel (inspection date order):  The south shore entrance (SFE) was on sill with a depth of 7.5 feet, on sill with a depth of 7.3 feet, and on sill with a depth of 8.5 feet.  The north powerhouse entrance (NFE) was on sill with a depth of 7.6 feet, on sill with a depth of 7.5 feet, and off sill with a depth of 8.0 feet.  The north shore entrance (NSE) was off sill with a depth of 5.8 feet, off sill with a depth of 6.0 feet, and on sill with a depth of 7.7 feet.  Fishway entrance criterion is 8 feet depth, greater than 8 feet depth, or on sill.  The north shore channel/tailwater differentials were 2.3 and 2.0 feet on August 10 and 11.  The shift operator adjusted the setting on the channel/tailwater differential.  All other channel/tailwater differentials were in criteria.  Channel/tailwater differential criteria are 1 – 2 feet.  On July 28, the sonar elevation detector failed at the north shore channel inspection location.  Currently, the north shore channel elevation is being measured by the old metra-tape system.  Replacement parts for the sonar equipment are on order.

### Auxiliary Water Supply System:  North shore fish pumps 1 and 2 were operated without any problems.  North shore fish pump 3 is on standby.  Six of the available 8 south fish pumps were operated without problem.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  No problems to report.  Fish ladder exits are clear of debris and the bubblers are operating.

STSs/VBSs:  STSs were switched to cycle mode operation on August 3 due to the size of sub-yearling Chinook being greater than 120 mm at Lower Monumental Fish Facility.  STS inspections for turbine units 1 and 2 are scheduled for August 18.  STS inspections for turbine units 3 and 4 are scheduled for August 25.

Orifices, Collection Channel, Dewatering Structure, and Bypass Pipe: 20 orifices are open.  The power supply wire system to the screen cleaner in the collection channel malfunctioned August 4.  The required parts have been ordered and are on the way.  The juvenile collection channel water elevations and the incline screen area continued to be monitored by power house operators and fish facility personnel.

Juvenile Bypass Facility:  The bypass is in operation with 20 orifices open providing 300 cfs into the juvenile collection channel.

Removable Spillway Weir:  The RSW is in the operating position.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Ice Harbor Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 34.4 | 31.4 | 24.3 | 21.2 | 71 | 70 | 8.0 | 7.4 |

*Unit 1 scrollcase temperature.

Other

Inline Cooling Water Strainers: Cooling water strainer gages are read 3 times a reporting period during each adult fishway inspection by fish facility personnel.

### Invasive Species: Zebra mussel monitoring devices are inspected by fish facility personnel during the first adult fishway inspection of the reporting period.  No mussels were found during this reporting period.

Project:  Lower Monumental Dam

# Biologists:  Bill Spurgeon and Ken Fone

Dates:  August 7 - 13, 2009

## Turbine Operation

All turbine units were operated within the 1% operation criteria.  Turbine unit 6 was taken out of service for annual maintenance on July 13 at 0700 hours and remained out of service through this time period.

Adult Fish Passage Facility

The adult fishway was inspected by Corps or state biologists on August 7, 8, and 12.

Fish Ladders: Fishway exit head differentials and depths over the weirs were within criteria (< 0.5’ and 1.0’-1.3’, respectively) on all inspections.  Picketed lead head differentials were in criteria (< 0.4’ and < 0.3’ for north and south shore fishways, respectively) on all inspections.

### Fishway Entrances and Collection Channel: NPE1 and NPE2 weir gates were in depth criteria (criteria: > 8’ or on sill) on all inspections.    North shore channel/tailwater head was in criteria (1’-2’) on all inspections.

SPE1 and SPE2 weir gates were in sill criteria (criteria: > 8’ or on sill) on all inspections, with an average depth of 5.4 feet.  South powerhouse channel/tailwater head was in criteria (1’-2’) on all inspections.

SSE1 weir gate was in sill criteria (criteria: > 8’ or on sill) on all inspections, with an average depth of 5.7 feet.  SSE2 was in criteria (6’ above sill) on all inspections.  South shore channel/tailwater head was in criteria (1’-2’) on all inspections.

### Auxiliary Water Supply System: AWS pump 3 has been out of service in support of bearing housing replacement and diffuser assembly repair.  Pumps 1 and 2 were in service throughout this period.  Pump 3 is “bulkheaded off” to minimize water loss during two-pump operation.  The average rpm of pumps 1 and 2 was increased from 71 to 74 on August 4 to increase the north collection channel velocity, which was 1.1 feet per second (criteria: 1.5-4.0 fps) on August 1.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  There was an average of 0 square yards of forebay debris observed during this period.  Gatewell debris ranged from 0 to 19% surface coverage.  No oil was observed in the gatewells.

STSs/VBSs: STS operation was changed to cycle mode on July 30 as the average fork length of sockeye and chinook juveniles exceeded the 120mm criteria.

Orifices, Collection Channel, Dewatering Structure, Flume: The bypass operated with 19 orifices open.   Channel, dewatering structure and flume functioned normally.

Collection Facility:  Fish are being collected for transport.  Equipment is functioning normally.

Transport Summary:  Barging ended on August 14 and alternate day midi-tanker transport then began.   The first load will depart on August 16.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Monumental Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature (oF)* |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 33.3 | 29.5 | 17.5 | 17.1 | 70 | 70 | 4.8 | 4.6 |

*Scrollcase temperatures.

#### Other

Inline Cooling Water Strainers:  The main unit cooling water strainers were cleaned on July 13.  Fifteen juvenile lamprey mortalities were recorded.  No other fish were present.  We plan to collect this information monthly through the year unless degradation of fish due to water temperature makes this task impossible.

Invasive Species: There were no zebra mussels observed at the monitoring stations on August 1.  The September inspection is expected to occur during the week of September 1.

Summer spill began on June 21.  Spill is scheduled to cease at the end of August.

Project:  Little Goose

# Biologist: Richard Weis

Dates:  August 7 - 13, 2009

## Turbine Operation

Turbine units 1, 2 and 4 through 6 were available for service during this report period.  Turbine unit 3 was returned to service on August 13 at 0854 hours following completion of annual maintenance.  All available turbine units were operated within 1% efficiency criteria.

## Adult Fish Passage Facility

USACE and ODFW fisheries biologists performed measured inspections of the adult fishway on August 9, 10 and 12.

Fish Ladder:  All criteria were met.

Fishway Entrances and Collection Channel:  Channel to tailwater head differentials ranged between 1.18 and 1.56 ft. (criteria 1.0 to 2.0 ft.).

SSE1 and 2 weir depths ranged between 8.14 and 8.28 feet (criteria ≥8.0 ft), NPE1 and 2 weirs rested on sill and depths ranged between 5.14 and 5.72 feet (criteria ≥7.0 ft. or on sill).  NSE1 weir depths ranged between 5.84 and 6.18 feet (criteria ≥ 6.0 ft.).  On August 12, NSE 2 measured below criteria at 5.84 feet.  NSE2 was measured within criteria on August 9 and 10, recording depths of 6.18 and 6.08 feet respectively (criteria ≥ 6.0 ft.).

Collection channel water velocities (criteria 1.5 to 4.0 fps) ranged between 2.1 and 2.7 fps near the SSE.  Subsurface water velocities near the SSE (flowmeter) ranged between 1.6 and 1.8 fps.

The fall-out fence was inspected this week.  No further deterioration was observed.

NPE3 (a concrete lift gate) which is normally closed, has deteriorated from water forces caused by spill over spillway 1.  Spill over spillway 1 forces collection channel water to remain within the channel and meet channel to tailwater head differentials criteria at fishway entrances. Repairs to NPE 3 are scheduled towards the end of the fish passage season.

Auxiliary Water Supply System:   All three fish pumps operated within criteria.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  Several pieces of floating forebay debris were observed inside the trash/shear boom.  Gatewells remained clear of debris and contaminants.

ESBSs/VBSs: ESBS operated within criteria this report period.  Video inspections of ESBS/VBS were conducted August 4 and 5.  All screens were found in proper operating condition.  The next scheduled video inspections are scheduled for October 6 - 8.  Trash rack cleaning is scheduled for August 17 -18.

Orifices, Collection Channel, Dewatering Structure, and Flume:  The collection channel was operated with 21 to 22 open orifices.  There were no orifice blockages during this report period.

The primary dewatering weir motor failed on July 20.  The motor is undergoing repairs.  Flows into the bypass system are regulated with the number of open orifices.

Transportation Facility:  Collection and transport continued.  Daily collection ranged between 117 and 865 for a total of 2,071 for the week.  Descaling was not observed this report period (i.e.: 0.0% descaling rate).  Daily smolt mortalities ranged between 1 and 15 for a totaled of 41 (2.0%).  The majority of mortalities appear to be disease (presumed columnaris) related.

Transport Summary:   Every-other-day barge departures continued.  All barge loading and transport operations were performed satisfactorily.

Spillway Weir:  No problems to report.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Little Goose Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 34.6 | 30.0 | 9.9 | 9.1 | 69.8 | 69.6 | 5.2 | 4.0 |

*Ladder temperature.

Other

Inline Cooling Water Strainers:  Cooling water strainers were checked on August 10.  Turbine units 1, 2, 4, 5, and 6 were inspected and no fish were observed.

Invasive Species:  The Zebra Mussel substrate was inspected on August 6.  No mussels were observed.   The next inspection is scheduled for September 2.

Up to six gulls were observed in the forebay during the report period.

Project:  Lower Granite

# Biologists: Mike Halter and Casey Watson

Dates:  August 7 - 13, 2009

## Turbine Operation

Lower Granite had turbine units 1, 2, 3, 4, and 5 available for power generation at the beginning of the report week.  Turbine unit 6 was taken out of service for annual maintenance on August 3 and remained out of service for the duration of the report week.  Doble testing of the 500 kv transmission line was conducted during much of the week and turbine unit 5 was operated in “speed no load” mode for station service only.  All water in excess of the 5 kcfs needed for station service was spilled.  Turbine units 1 through 4 were taken out of service in support of Doble tests on August 10 (0640 hours to 1735 hours), August 11 (0619 hours to 1735 hours), August 12 (0557 hours to 1812 hours) and August 13 (0559 hours to 1759 hours).

## Adult Fish Passage Facility

On August 7, 8, 10, and 11 the Lower Granite fishery biologists performed measured inspections of the adult fishway system.

Fish Ladder:  All criteria were met during the week.  The ladder weir depth at diffuser 14 has met criteria since the adult trap was taken out of service.

Fishway Entrances and Collection Channel:  Head differential readings remained within criteria at all adult fishway entrances during the weekly inspections.  Weir depths at the south shore fishway entrances were within criteria whenever the tailwater depths were sufficient to keep the gates off their seals.  (At tailwater elevations below 633.0 feet the south shore gates bottom out).  Weir depths at the south shore entrances ranged between 7.3 and 8.3 feet during the weekly inspections (criterion ≥ 8.0 feet).  Weir depths at the north powerhouse fishway entrances were below the desired 8.0 foot criteria level on all inspections this week due to low tailwater elevations which caused the gates to bottom out and rest on their sills.  (At elevations below 636.0 feet the north powerhouse gates bottom out.)  North powerhouse entrance weir depths ranged from 4.5 to 5.4 feet.  Weir depth readings at the north shore fishway entrances were out of criteria all week and ranged between 5.0 and 5.6 feet (criterion ≥ 7.0 feet).  Weir depth readings at the north shore entrances continue to be sacrificed in order to maintain the requisite 1.0 foot of head differential.  Weekly velocity readings in the adult fishway collection channel transition pool area ranged from 0.86 to 1.20 feet per second and averaged 1.00 feet per second.

### Auxiliary Water Supply System:  Fish pumps 1 and 3 were run during the week without any problems.  The bearing work on fish pump 2 has been completed.  Fish pump 2 testing will be conducted in the near future.

#### Juvenile Fish Passage Facility

The facility sample rate remained at 50% during the report week.

Forebay Debris/Gatewell Debris/Oil:   Debris levels remained very low during the week.  There were no plugging issues of any kind.

ESBSs/VBSs:  The last video camera inspections of the ESBSs and VBSs were completed on June 19.  The next video inspections are scheduled for August.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:   As a fail-safe, collection gallery orifices were back-flushed every three hours throughout the week.

Transportation Facility:  Every-other-day fish barging operations continued during the week at Lower Granite, Little Goose, and Lower Monumental dams as did daily barge operations at McNary Dam.  A second barge operation is taking place at McNary Dam to barge fish on the days the Lower Granite barge is not available.

Transport Summary:   All operations continue to go smoothly.  Fish barges are departing Lower Granite Dam on the even numbered days during the month of August.  The last fish barge is scheduled to depart Lower Granite on July 14.  Fish trucking operations are scheduled to begin on August 16.

Removable Spillway Weir:   Nothing to report on the RSW.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Granite Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 34.6 | 31.5 | 24.4 | 18.6 | 68.5 | 64.6 | 5.0 | 5.0 |

*Scrollcase temperature.

Other

Inline Cooling Water Strainers:   Inline cooling water strainers on turbine unit 4 were last checked for lamprey on July 14.  One lamprey (alive) was found in one strainer basket.  The other basket had evidence of 3 dead lamprey.

Invasive Species:  No evidence of zebra or quagga mussels was found during the week.  The zebra mussel detector at the head of the adult fishway was last checked on August 11.

Adult fish trap operations were discontinued on July 19 due to problems with the air valve control for the slide gate to the anesthetic tank and also due to high water temperatures.  Trap operations are tentatively scheduled to resume on August 18 if water temperatures are low enough to begin work.  The goal at the adult trap this year is to maintain a 12% sample rate through the fall season.  In addition, a goal of 2,000 unclipped (wild) steelhead are to be PIT-tagged for Idaho Fish and Game.