U.S. ARMY CORPS OF ENGINEERS

WALLA WALLA DISTRICT

#### FISH FACILITIES WEEKLY REPORT

#43-2009

# Project:  McNary

# Biologists:  Brad Eby and Bobby Johnson

Dates:  December 18 - 24, 2009

## Turbine Operation

McNary had 13 to 14 turbine units available for power generation this week.  From December 21 to 23, turbine unit 4 was out of service due to inoperative ESBSs in slots 4A and 4C.  On December 22 and 23, turbine unit 3 was out of service for a total of 7.3 hours in support of portable crane work at turbine unit 4.  ESBS removals in this location for winter maintenance also required the shut down of transmission line 2.  All available turbine units continued to operate within the soft one percent constraint limitations.

## Adult Fish Passage Facilities

On December 21 and 23, the McNary fisheries biologists performed measured inspections of both adult fishways.

Fish Ladders:  During measured inspections, both ladders met all Fish Passage Plan criteria except on December 23 when the head over weir differentials measured 1.4 feet and 1.7 feet respectively at the Oregon and Washington ladders.  The Oregon ladder continues to have problems with the encoders (control system) at weirs 337 and 340.  On December 21, the Washington ladder exit tilting weirs 336 and 337 along with the associated PLC (Programmable Logic Circuits – electronic controls) began to experience flow control problems, triggering alarms repeatedly.  Both exit weirs will be operated manually until the control issues are resolved.  The remaining Washington ladder tilting weirs are in automatic operation.  On December 22, a tripped electrical breaker as discovered at the Washington exit.  Normal exit weir operation was restored after the breaker was reset.  The Washington exit weirs also underwent scheduled maintenance this week.  The ladder exit traveling screens remains out of service for maintenance and ice damage prevention.

Fishway Entrances and Collection Channel:  All Washington ladder inspection points measured in criteria.  All Oregon ladder inspection points were in criteria except for north and south pool differentials.  North pool differentials measured 0.5 and 0.1 feet and south pool differentials measured 0.8 and 0.1 feet.  The 0.1 feet readings occurred on December 23 when only one fish pump was in use.  The other differential readings are probably influenced by the operation of the juvenile fishway in emergency bypass mode and closure of the 1000 cfs auxiliary water conduit.  No formal collection channel velocity measurements were taken this week, but the velocity appears to be approximately 1.0 fps.

Auxiliary Water Supply System:  All three fish pumps operated through most of this week with blade angles of 22 to 30 degrees.  On December 22 and 23, pumps 1 and 3 were out of service 7.5 hours in support of transmission line 2 outages mentioned above.  The 1,000 cfs conduit remains closed for winter maintenance, traveling screen repair and for the prevention of winter freeze damage.  The juvenile facility remains in emergency bypass and is no longer supplying the usual 450 cfs to the north powerhouse junction pool.  From December 19 to 21, the Wasco County PUD unit tripped off line on 3 occasions with outage times totaling four hours.  The bypass-energy dissipater operated well during these outages.

#### Juvenile Fish Passage Facility

The juvenile system remains in emergency bypass.  This will continue until all ESBSs are raised for maintenance later this month, at which time the channel orifices will be closed.

Forebay Debris/Gatewell Debris/Oil:  Forebay debris remains light.  Trash rack differentials remained low and no racks were cleaned this week.  Slots 9B, 9C, 10A, 10B, 14A, 14B and 14C remain covered in support of contract work on the intake deck crane’s main hoist and for portable crane parking.  Since the intake deck crane’s main hoist remains out of service, some work may be delayed.  There were no other problems observed other than slight oil sheens being reoved from slots 4B and 4C with absorbent pads.

ESBSs/VBSs:  ESBSs are currently deployed in all turbine units.  The screens in slots 9C and 11C remain in transducer bypass mode.  On December 21, ESBSs in slots 4A and 4C were found inoperative after being switched to automatic mode.  The turbine unit was taken out of service and all three associated screens were subsequently raised on December 22 and 23.  This turbine unit had mostly been in standby mode since being returned to service on December 16.  On December 23, the screen in slot 5C triggered an alarm and returned to service after being placed into bypass mode.  ESBS camera inspections did not occur this week as all screens will be physically inspected in the near future during winter maintenance.  VBS monitoring revealed no screens out of criteria and none were cleaned this week.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:  For the week, there were 42 orifices opened.  No problems occurred and all channel systems are out of service for winter maintenance.  Water was noted in the air lines and this issue will be addressed in the near future.

Transportation Facility:  Emergency bypass operations continued.  The facility was previously dewatered and winterized.  The winter maintenance season continues.

Transport Summary:  There is nothing to report.

River Conditions

River conditions during the week are outlined in Table 1 with data provided by the control room whose data day runs from 0000 to 2400 hours.  Scheduled and contracted maintenance on spillbay hoists and gates continued.  On December 18 and 22, slight spill occurred at bays 5 and 8 during gate tests.  The bays were opened to three foot openings and closed three times in 20 minutes.  The effect of this activity on daily spill average spill calculations was minimal.

Table 1.  River conditions at McNary Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 118.8 | 84.3 | 0.0 | 0.0 | 42.0 | 40.0 | 6.0 | 6.0 |

Other

Inline Cooling Water Strainers:  Cooling water strainers were not inspected this week.

Invasive Species:  The zebra mussel stations were last examined December 14 and no mussels were found.

During casual predatory bird observations of the tailwater area, gulls, mergansers and cormorants were observed roosting near or feeding in the powerhouse flow with an occasional kingfisher being noted.  On December 23, three bald eagles and seven pelicans were observed.  Some birds were seen at the emergency bypass outfall.  In forebay locations, only an occasional gull, cormorant or grebe was noted.  No grebes were noted elsewhere within the facilities.  Bird numbers appear to be fluctuating with the juvenile shad out-migrations and bird winter migrations.

Project:  Ice Harbor

# Biologist:  Mark Plummer

# Technician: Stephen Jeffers

Dates:  December 18 - 24, 2009

## Turbine Operation

Turbine units 2 – 6 were available for operation the entire reporting period.  Turbine unit 1 remains out of service for overhaul.

## Adult Fish Passage Facilities

### Fish facility personnel inspected the adult fishways on December 21, 22, and 23.

### Fish Ladders:  All north and south fish ladder inspection areas (picketed leads, head differentials, fishway exits, and depth over weirs) were within criteria.  The north shore adult fish ladder is tentatively scheduled to be unwatered to tailwater elevation for annual maintenance and inspection from January 4 to January 14.   The south shore adult fish ladder is tentatively scheduled to be unwatered completely for annual maintenance and inspection from January 19 to February 25.

### Fishway Entrances and Collection Channel (inspection date order):  The south shore entrance (SFE) was on sill with a depth of 6.8 feet, on sill with a depth of 7.3 feet, and on sill with a depth of 7.9 feet.  The north powerhouse entrance (NFE) was on sill with a depth of 6.8 feet, on sill with a depth of 7.4 feet, and on sill with a depth of 8.0 feet.  The north shore entrance (NSE) was on sill with a depth of 6.7 feet, on sill with a depth of 7.3 feet, and on sill with a depth of 7.8 feet.  Fishway entrance criterion is 8 feet depth, greater than 8 feet depth, or on sill.  All channel/tailwater differentials were within criteria of 1 – 2 feet.

### Auxiliary Water Supply System:  North shore fish pumps 1 and 2 were operated without any problems.  North shore fish pump 3 went out of service December 14 to diagnose and correct a small oil leak thought to be located around the pump shaft.  All 8 south fish pumps are available for operation.  Currently, 6 of the available 8 south fish pumps were operated without any problems.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  No problems to report.  Fish ladder exits are clear of debris and the bubblers are operating satisfactorily.

STSs/VBSs: All STSs have been raised for annual maintenance.  STSs in turbine units 1 and 6 were respectively raised December 3 and December 14.  The remaining STSs were raised for winter maintenance on December 16 and 17.

Orifices, Collection Channel, Dewatering Structure, and Bypass Pipe: The collection channel, dewatering structure, and bypass pipe were dewatered on December 17.  The power supply system to the JFF screen cleaner which is located in the juvenile collection channel has not worked properly for some time.  Due to the hazards of working over the operating juvenile fish channel when watered, permanent repairs were delayed until the winter maintenance period.

Juvenile Bypass Facility:  The juvenile fish bypass was unwatered for winter maintenance on December 17.

Removable Spillway Weir:  The RSW weir is in the deployed position.  Currently, the RSW weir is not in operation as the fish passage spill season ended August 31.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Ice Harbor Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 25.1 | 16.8 | 0 | 0 | 41 | 40 | 9.0 | 8.6 |

*Unit 1 scrollcase temperature.

Other

Inline Cooling Water Strainers: Cooling water strainer pressures are recorded 3 times a reporting period during each adult fishway inspection performed by fish facility personnel.

### Invasive Species: Zebra mussel monitoring devices are inspected by fish facility personnel during the first adult fishway inspection of the reporting period.  No mussels were found during this reporting period.

Project:  Lower Monumental Dam

# Biologists:  Bill Spurgeon and Ken Fone

Dates:  December 18 - 24, 2009

## Turbine Operation

All turbine units were operated within the 1% operation criteria.  Turbine unit 5 was taken out of service for annual maintenance on November 30 at 0700 hours and remained inoperative throughout this period.

Adult Fish Passage Facility

The adult fishway was inspected by corps biologists on December 21 and 22.

Fish Ladders: Fishway exit head differentials and depths over the weirs were within criteria (< 0.5’ and 1.0’-1.3’, respectively) on all inspections.  Picketed lead head differentials were in criteria (< 0.4’ and < 0.3’ for north and south shore fishways, respectively).

### Fishway Entrances and Collection Channel: NPE1 and NPE2 weir gates were in depth criteria (criteria: > 8’ or on sill) on all inspections.  North shore channel/tailwater head was in criteria (1’-2’) on all inspections.

SPE1 and SPE2 weir gates were in depth or sill criteria (criteria: > 8’ or on sill) on all inspections, with an average on sill depth of 7.4 feet.  South powerhouse channel/tailwater head was in criteria (1’-2’) on all inspections.

SSE1 weir gate was in depth criteria (criteria: > 8’ or on sill) on all inspections.  SSE2 was in criteria (6’ above sill) on all inspections.  South shore channel/tailwater head was in criteria (1’-2’) on all inspections.

### Auxiliary Water Supply System: AWS pump 3 remains out of service for bearing housing replacement and diffuser assembly repair.   Pump 3 is “bulkheaded off” to minimize water loss during two-pump operation.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  There was an average of 105 square yards of forebay debris observed during this period.  Gatewell debris can not be seen as STS screens are presently raised for winter maintenance.  No oil was observed in the gatewells.

STSs/VBSs:  STSs were raised for winter maintenance on December 16 and 17.

Orifices, Collection Channel, Dewatering Structure, Flume:  The bypass system was dewatered for winter maintenance on December 16.

Collection Facility:  Currently undergoing winter maintenance.

Transport Summary:  Transport ended October 1.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Monumental Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature (oF)* |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 26.0 | 17.3 | 0 | 0 | 39.5 | 39.5 | 5+ | 5+ |

*Scrollcase temperatures.

#### Other

Inline Cooling Water Strainers: Unit cooling water strainers were checked on December 3.  Fish were not counted as fisheries personnel were unavailable.

Invasive Species: There were no zebra mussels observed at the monitoring stations on December 3.

Due to reports of upcoming freezing weather, the bird abatement sprinklers were shut off on December 3.

Project:  Little Goose

# Biologist: Rick Weis

Dates:  December 18 - 24, 2009

## Turbine Operation

Turbine units 1, 3, 4, 5 and 6 were available for power generation throughout this report period. Turbine unit 2 was forced out of service on October 1 due to thrust oil pump failure - repairs are ongoing.  All available turbine units operated within 1% efficiency criteria this week.

## Adult Fish Passage Facility

COE fisheries biologists performed measured inspections of the adult fishway on Dec. 21 and 22.

Fish Ladder:  All criteria were met.

Fishway Entrances and Collection Channel:  Channel to tailwater head differentials ranged from 1.08 to 1.20 feet near the SSE; 0.84 to 0.98 feet near the NPE and 0.86 feet for both inspections near the NSE (criteria 1.0 to 2.0 ft.).

SSE1 and 2 weir depths ranged from 8.28 to 9.14 feet (criteria ≥8.0 ft).  NPE1 and 2 weir depths ranged between 7.14 (sill) and 7.24 feet (criteria ≥7.0 ft. or on sill).  NSE1 weir previously broke free from the operator and is resting on sill at 529.0 ft.  NSE 1 weir depths ranged between 7.54 and 7.80 feet (criteria ≥ 6.0 ft).  NSE2 was completely raised and closed off on November 16 at 0940 hours to compensate for water loss through damaged NPE3 and NSE1.

Collection channel surface water velocities (criteria 1.5 to 4.0 fps) ranged between 2.4 and 2.8 fps near the SSE.  Subsurface water velocities near the SSE (flowmeter) ranged between 2.3 and 2.4 fps (criteria 1.5 to 4.0 fps).

Auxiliary Water Supply System:    All three fish pumps operated within criteria.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  Forebay debris remained minimal.  Gatewells for the most part remained clear of debris and contaminants.

ESBSs/VBSs:  All ESBS screens were previously removed for winter maintenance.

Orifices, Collection Channel, Dewatering Structure, and Flume:  The juvenile collection channel and bypass flume was dewatered for the season on December 16.

Transportation Facility:  Facility operations were switched to primary bypass mode on October 31.  The water supply/emergency bypass pipe and the facility were dewatered on November 3.  Winter maintenance is in progress.

Transport Summary:  Transport operations ended October 31.

Spillway Weir:  Spillway 1 and the TSW were removed from service on August 31.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Little Goose Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 21.6 | 17.7 | 0 | 0 | 38.9 | 38.7 | 6.0+ | 6.0 |

*Ladder temperature.

Other

Inline Cooling Water Strainers:  Cooling water strainers were not inspected this week.

Invasive Species:  The Zebra Mussel substrate was inspected on December 14.  No mussels were observed.   The next inspection is scheduled for January 16.

Up to 21 gulls and 6 cormorants were observed in the vicinity of the dam during this report period.

Project:  Lower Granite

# Biologist: Mike Halter

Dates:  December 18 - 24, 2009

## Turbine Operation

Lower Granite had turbine units 1 - 6 available for power generation during the report week.  All operating turbine units were run within the established guidelines.

## Adult Fish Passage Facility

On December 21, 22, and 23 the Lower Granite fisheries biologist performed measured inspections of the adult fishway system.

Fish Ladder:  All criteria were met during the week.

Fishway Entrances and Collection Channel:  Head differential readings remained within criteria at all adult fishway entrances during the weekly inspections.  Weir depths at the south shore entrance #2 remained within criteria all week.  Weir depths at south shore entrance #1 met criteria on the December 21 inspection but fell slightly below criteria on the December 22 and 23 inspections with readings of 7.9 feet (criterion ≥ 8.0 feet).  Weir depths at the north powerhouse entrances met criteria on all inspections this week. Weir depths at the north shore fishway entrances were below criteria on all inspections this week and ranged from 5.0 to 5.1 feet (criterion ≥ 7.0 feet). Weir depth readings at the north shore entrances continue to be sacrificed in order to maintain the requisite 1.0 foot of head differential. Weekly velocity readings in the adult fishway collection channel transition pool area ranged from 0.88 to 1.03 feet per second and averaged 0.96 feet per second.

### Auxiliary Water Supply System:  On December 21, a bulkhead was put into place to seal off fish pump 1 and fish pump 2 was placed into service.  Some breaker problems developed during the switch and the process took longer than anticipated.  Fish pump 3 (only) was operated from 1045 hours to 1309 hours while the switch was taking place.  Following the switch, fish pumps 2 and 3 were run for the duration of the week without any problems.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  Debris levels were light during the week. There were no plugging issues of any kind.

ESBSs/VBSs:  All ESBSs have been raised and dogged off for the winter maintenance season.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:  The juvenile fish collection gallery has been dewatered for the winter maintenance season.

Transportation Facility:  The juvenile fish collection gallery and collection/transportation facility were dewatered for the season on the morning of December 16.

Transport Summary:  Nothing to report.  Fish transportation activities have concluded for the season.

Removable Spillway Weir:  Dive work was conducted during the week on the control mechanisms that regulate the lowering of the structure to the resting pad on the river bottom.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Granite Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 22.7 | 19.7 | 0.0 | 0.0 | 45.5 | 38.5 | 5.0 | 5.0 |

*Scrollcase temperature.

Other

Inline Cooling Water Strainers:   Inline cooling water strainers on turbine unit 3 were checked for lamprey on September 28.  No evidence of live or dead lamprey were sighted.

Invasive Species:  No evidence of zebra or quagga mussels was found during the week.  The zebra mussel detector at the head of the adult fishway was last checked on December 15.

Adult Fish Trap:  The adult fish trap complex remains dewatered for the winter season.