U.S. ARMY CORPS OF ENGINEERS

WALLA WALLA DISTRICT

#### FISH FACILITIES WEEKLY REPORT

#26-2010

# Project:  McNary

# Biologists:  Brad Eby and Bobby Johnson

Dates:  August 20 - 26, 2010

## Turbine Operation

McNary had eight to ten turbine units available for power generation this week.  Turbine units 2 and 7 are expected to remain out of service for rewind work until the end of the year.  Turbine units 3 and 4 remain out of service in support of T2 transformer replacement.  Transformer replacement was completed August 26.  Turbine units 3 and 4 were briefly test-operated after the completion of transformer work.  Both turbine units are expected to return to full service early next week.  On August 21, turbine units 10 and 11 went out of service after respectively experiencing VBS and ESBS problems.  Turbine unit 10 returned to service after 6.2 hours following VBS replacement in slot 10A.  Turbine unit 11 returned to service August 23 following ESBS replacement in slot 11B.   From August 23 to 26, turbine unit 9 was out of service for annual maintenance.  All turbine units operated within the 1% peak efficiency criteria this week.

## Adult Fish Passage Facilities

McNary fishery biologists performed measured inspections of the adult fishways on August 21, 25 and 26.  Adult fish counts continued.  Oregon entrance weirs SFEW1, SFEW2 and NFEW1 are being lowered during nighttime hours on a predetermined schedule as part of the adult lamprey passage study.  On August 26, attempts were made to raise the picketed leads 1.5 inches in support of the passage study, but these attempts failed.  Otherwise, no problems have been observed.  The leads will need to be modified during winter maintenance.  Ladder exit temperature monitoring continues.

Fish Ladders:  Both ladders met all Fish Passage Plan criteria during measured inspections.  The regulating weir near the Washington exit continues to occasionally trigger alarms when adjusting tilt angles.  In each case, normal operation resumed after the alarm was reset.  On August 26, the Oregon picketed leads were cleaned after milfoil accumulations caused water to overtop the leads.

The Oregon exit’s regulating weir and tilting weir 340 triggered an alarm once this week due to encoder problems.  The encoder is a device that measures weir movement.  Normal operation resumed after the alarm was reset.  The north Oregon exit traveling screen remains out of service. The south traveling screen continues to operate well and daily differential monitoring efforts revealed no problems.

Fishway Entrances and Collection Channel:  All Washington ladder inspection points were in criteria except on August 25 when entrance weir W3 measured 7.0 feet.  The weir was calibrated the following day.  All Oregon ladder inspection points met criteria except for the north powerhouse pool differential which measured 0.5 to 0.8 feet.  Low tailwater and hydraulic gradients continue to affect measurements.  Collection channel velocities averaged 1.5 feet per second.

### Auxiliary Water Supply System:  Fish pumps 2 and 3 ran satisfactorily this week with blade angles of 30 degrees and 3 brief interruptions in service. A faulty bearing temperature sensor caused fish pump 3 to trip beakers twice on August 21.  Outage times totaled 1.3 hours.  Fish pumps 2 and 3 were briefly taken out of service on August 23 to allow removal of stoplogs associated with fish pump 1.   Stoplogs are to be refurbished prior to the start of the winter maintenance season in December.  Fish pump 1 remains out of service as only two pumps can be operated with transformer 2 being unavailable.  The juvenile facility continued to supply the usual 450 cfs to the north powerhouse pool and the Wasco County PUD turbine unit had one unscheduled 1.1 hour interruption in service during this report period for unknown reasons.  The bypass operated well during the outage.

#### Juvenile Fish Passage Facility

Collection for truck transport continued with every other day truck departures.  For the report week, 35,175 smolts and 150 juvenile lamprey were collected.

Forebay Debris/Gate well Debris/Oil:  Forebay debris was very light and consisted mostly of Eurasian milfoil.  Trash rack differentials were satisfactory this week and no racks were cleaned.  Slots 3B, 3C, 4A and 4B slots remain covered in support of transformer replacement.  A slight oil sheens was removed from one slot with absorbent pads.  No other problems were observed.

ESBSs/VBSs:  ESBSs are currently deployed in all units and screens in slots 5C, 6A, 9C and 10C remain in transducer bypass mode.  No camera inspections occurred this week.  A failed screen in slot 11B was replaced on August 23.  VBS monitoring revealed no screens out of criteria and cleaning was not necessary.  As part of an ongoing preventative maintenance program, the VBS in slot 10A was replaced with a rehabilitated screen on August 21.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:  Thirty-nine to forty orifices were in use all through the week as forebay elevations remain fairly stable.  The orifices associated with turbine units 2 and 3 (which are out of service) are opened or closed as required when forebay elevations are high.  On August 25, the remaining closed orifice associated with turbine unit 3 was opened as this turbine unit is expected to return to service next week.  All systems operated well in automatic mode.

Transportation Facility:  Alternating days of truck transport continued.  Primary (A and B) PIT tag slide gates are currently in service and PIT tagged fish are being routed directly to the river.  The secondary (C and D) PIT/bypass gates remain closed and off for fish collection.  Sample gates continued to function well.  Sample rates were changed to 20% on August 20, followed by the initiation of PIT tag gate override on August 21.  The override allows PIT tag diversion gates to operated when the sample gate is open.  This mode is required whenever sampling rates exceed 20%.  Daily fish mortality rates ranged from 0.25 to 1.73 percent for the report period.

As mentioned in the previous weekly report, facility personnel noted problems with shad weight and count data on August 19.  The shad data problems were completely resolved by August 23.

Transport Summary:  Trucks left McNary on August 20, 22, 24, 25 and 26.  The August 25th trip was added due to high numbers of shad.  Because of their small size and flat body shapes juvenile shad can clog screens, drains and circulation systems in trucks and raceways.  Adding a trip lowers fish density and reduces the likelihood of transport trailer circulation system problems.  On August 24, the trailer pump indicator light was repaired.  Otherwise, transport operations proceeded normally.

River Conditions

River conditions during the week are outlined in Table 1 as provided by the smolt monitoring staff whose data day runs from 0700 to 0700 hours each day.  The summer spill program continues with approximately 50 percent of the river discharge being spilled.  Spill patterns were adjusted for brief periods to allow safe navigation.  WDFW personnel continue to monitor water temperatures very closely, and no problems have been observed.  Water temperature monitoring results are being reported separately.

Table 1.  River conditions at McNary Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature (oF) |  | Water Clarity* (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 132.0 | 107.4 | 66.1 | 53.8 | 70.4 | 69.0 | 6.0 | 6.0 |

*Control room data.

Other

Inline Cooling Water Strainers:  Cooling water strainer inspections last took place on August 18 in turbine unit 6.  No fish were noted.

Invasive Species: No invasive species were found during the August 1 detector inspection.  The next examination will occur September 1.

Predatory bird counts continued with tailwater and forebay observations conducted twice and once daily, respectively.  The tailwater high counts included 50 gulls, 35 cormorants and 5 terns.  Most tailwater birds were seen in the spill basin.  Overall bird numbers appeared to be fluctuating, and no birds were seen around the bypass outfalls.  Forebay area high counts included an occasional grebe, gull or osprey being noted.  The three grebes noted in the collection channel at the end of the previous report period are still present.  No other grebes entered the collection system this week.  The water cannon continued to operate satisfactory.  The adult lamprey passage studies continued as did GBT monitoring efforts.

Project:  Ice Harbor

# Biologist: Mark F. Plummer

Dates:  August 20 - 26, 2010

## Turbine Operation

Turbine units 1- 6 were available for operation during this reporting period.  Turbine unit 6 returned to service from maintenance on August 20 at 1310 hours.  Turbine unit 6 again went out of service August 23 at 0638 hours for annual maintenance.

## Adult Fish Passage Facilities

### Fish facility personnel inspected the adult fishways on August 20, 23, and 26.

### Fish Ladders:  All north and south fish ladder inspection areas (picketed leads, head differentials, fishway exits, and depth over weirs) were within criteria.  Aquatic vegetation growth accumulations on the south picketed leads have decreased, allowing a reduction in cleaning frequency.

### Fishway Entrances and Collection Channel (inspection date order):  The south shore entrance (SFE) was on sill with a depth of 7.3 feet, on sill with a depth of 6.2 feet, and on sill with a depth of 6.7 feet.  The north powerhouse entrance (NFE) was on sill with a depth of 7.7 feet, on sill with a depth of 6.2 feet, and on sill with a depth of 6.7 feet.  The north shore entrance (NSE) was on sill with a depth of 5.7 feet, on sill with a depth of 6.2 feet, and on sill with a depth of 5.2 feet.  Fishway entrance criterion is 8 feet depth, greater than 8 feet depth, or on sill.  The north fish entrance and the north powerhouse fish entrance are being operated in manual mode as spill causes constant weir adjustments and jammed entrances.  All out of criteria depths were reported to the operator for corrections.  The north shore channel/tailwater readings on August 20 and 26 were above the criteria of 2.0 feet. Readings on August 20 and 26 were respectively 2.8 feet and 3.0 feet.  All other channel/tailwater differentials were in criteria.  Spill continues to make visual staff gage reading difficult due to rapid changes in water elevation at the staff gages.  Visual staff gage readings are noted in this report.

### Auxiliary Water Supply System:  Two of the 3 north shore fish pumps were operated without problems.  All north shore fish pumps are in service.  Six of the 8 south shore fish pumps were operated.  South fish pump 4 went out of service July 23rd for repairs after personnel noticed excessive motor bearing noises.  Currently, 5 of the 8 south fish pumps are being operated to facilitate fish pump motor replacements.  The south adult fishway is currently able to stay in criteria with only 5 south fish pumps in service.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  There is minimal forebay debris.  Fish ladder exits are clear of debris and the bubblers are operating satisfactorily.

STSs/VBSs:  STSs are being operated in continuous mode.  No problems were found during the August STS/VBS inspections.  September STS/VBS inspections are scheduled for the 13th and 15th.

Orifices, Collection Channel, Dewatering Structure, and Bypass Pipe:   The juvenile fish bypass is in operation.  Twenty orifices are open.

Juvenile Bypass Facility:  The bypass was placed in operation on March 8, 2010.

Juvenile Fish Sampling:  Sampling for the 2010 season concluded July 1 when the number of juvenile fish sampled during a 4 hour period dropped below 100 on 2 consecutive sample dates.

Removable Spillway Weir:  RSW operation continues.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Ice Harbor Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 32.5 | 26.4 | 22.2 | 16.4 | 70 | 70 | 5.9 | 5.4 |

*Unit 1 scrollcase temperature.

Other

Inline Cooling Water Strainers:  Cooling water strainer cleanings took place in turbine units 4, 5 and 6 on August 9 and in turbine units 1, 2 and 3 on August 11.  No lampreys were found.

Invasive Species: No zebra mussels were observed during inspections on August 20.

# Project:  Lower Monumental Dam

# Biologist(s):  William Spurgeon and Elizabeth Lindsey

Dates:  August 20 - 26, 2010

## Turbine Operation

Turbine units are being operated in hard constraint 1% operational criteria.  Turbine unit 2 was previously taken out of service for annual maintenance on August 2 at 0809 hours.  Turbine unit 5 remains out of service as of August 10 at 0957 hours pending contractor evaluation.

Adult Fish Passage Facility

The adult fishway was inspected by Corps or State biologists on August 20, 21, 22, and 25.

Fish Ladders: Fishway exit head differential and depth over the weirs were within criteria (< 0.5’ and 1.0’-1.3’, respectively) on all inspections.  Picketed lead head differentials were in criteria (< 0.4’ and < 0.3’ for north and south shore fishways, respectively) on all inspections.

### Fishway Entrances and Collection Channel: NPE 1 and NPE 2 weir gates were in depth criteria (criteria: > 8’ or on sill) on all inspections.  North shore channel/tailwater head was in criteria (1’-2’) on all inspections.

SPE 1 and SPE 2 weir gates were in sill criteria (criteria: > 8’ or on sill) on all inspections.  While on sill, the gate depth readings were 5.7’, 6.0’, 5.6’, and 5.5 feet.  South powerhouse channel/tailwater head was in criteria (1’-2’).

SSE1 weir gate was in sill criteria (criteria: > 8’ or on sill) on all inspections.  While on sill, the gate depth readings were 6.0’, 6.5’, 5.9’, and 6.1 feet.  SSE2 was in criteria (6’ above sill) on all inspections.  South shore channel/tailwater head was in criteria (1’-2’).

### Auxiliary Water Supply System: AWS pump 3 remains out of service in support of bearing housing and diffuser assembly repairs.  This work is scheduled to be completed in September.  Pumps 1 and 2 were in service throughout this period.  Pump 3 remains “bulkheaded-off” to minimize water loss during two-pump operation.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  There was an average of 0.0 square yards of forebay debris observed during this period.  Gatewell debris surface coverage ranged from 0-22 percent.  No oil was observed in the gatewells.

STSs/VBSs:  STS operation was changed to cycle mode on August 10 as average fork length of sub-yearling chinook and/or sockeye exceeded 120mm.  STSs inspections were conducted on August 2 and 3.  All screens passed inspection.

Orifices, Collection Channel, Dewatering Structure, Flume:  The collection channel is operating with 19 orifices open.

Collection Facility:  No problems were observed in facility systems or operation.

Transport Summary:  Every other day transport by midi-tanker is in progress.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Monumental Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature (oF)* |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 32.3 | 24.6 | 17.4 | 12.3 | 69.0 | 68.0 | 4.8 | 4.2 |

*Scrollcase temperatures.

#### Other

Inline Cooling Water Strainers:  Cooling water strainers were inspected August 2.  Two lamprey mortalities were recovered.  Eighteen Siberian prawns were also found.

Invasive Species:  There were no zebra mussels observed at the South shore monitoring stations on August 8.

Summer spill operation was initiated at 0003 hours on June 21.  The average spill for this period was 16.3 kcfs.

Project:  Little Goose

# Biologist:  George Melanson

Dates:  August 20 - 26, 2010

## Turbine Operation

Turbine units 2 through 6 were available for power generation throughout this reporting period.  Turbine unit 1 was removed from service on August 16 for annual maintenance. There were no reports of turbine units operating outside the 1% of best efficiency.

## Adult Fish Passage Facility

COE and ODFW fisheries biologists performed measured inspections of the adult fishway on August 20, 24 and 26.

Fish Ladder:  The ladder exit head differential ranged between 0.0 and 0.2 feet (criteria < 0.5 ft.).  Water depths over the weirs held steady measuring 1.1 feet (criteria 1.0-1.3 ft.) and picketed lead head differentials held steady a 0.0 feet (criteria < 0.3 ft.).

Fishway Entrances and Collection Channel:  Channel to tailwater head differentials at fish entrances ranged between 1.06 and 1.64 (criteria 1.0 to 2.0 ft.).  SSE weir depths ranged between 8.20 and 8.26 feet (criteria ≥8.0 ft), NPE1 and 2 weirs rested on sill position and depths ranged between 5.14 and 5.46 (criteria ≥7.0 ft. or on sill).  NSE1 is manually positioned at 529.1 and depths measured between 7.72 and 8.32 feet (criteria ≥ 6.0 ft.).  NSE2 weir depth ranged between 6.16 and 6.18 feet (criteria ≥ 6.0 ft.).

Collection channel surface water velocities (criteria 1.5 to 4.0 fps) ranged between 1.6 and 1.9 fps near the SSE and 2.4 and 2.9 fps near the NSE.   Subsurface water velocities near the SSE (flowmeter) ranged between 0.7 and 1.0 fps.

Auxiliary Water Supply System:  All fish pumps operated within criteria during this report period.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  Surface debris in the immediate forebay remained minimal during this report period.  Gatewells remain clear of debris.

Spillway Weir:  The weir was removed from service on August 5.  Summer spill continues.

ESBS/VBS:   All operating ESBSs were within criteria this report period.

Orifices, Collection Channel, Dewatering Structure, Flume:  The juvenile collection channel continued to operate with 23 open orifices – no problems to report.

Transportation Facility:  Collection and transport continued this report period.  Daily collection for the period ranged between 76 and 873 for a total of 1,669 for the week.  All fish were transported except for mortalities.  Daily descaling rates ranged between 0.0% and 0.8% while daily mortality rates averaged 1.4% for this report period.

Transport Summary:  Every-other-day truck transport operations continued.  Salt in small concentrations of approximately 1 g/L is added to the midi-tank water to treat potential Columnaris disease and to reduce stress.  All transport operations were successfully completed without incident.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Little Goose Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 33.4 | 25.9 | 11..2 | 9.3 | 69.8 | 68.5 | 5.8 | 5.1 |

*Ladder temperature.

Other

Inline Cooling Water Strainers:  Cooling Water Strainers were inspected on August 26.  No fish or fish remains were found.

Invasive Species:  The Zebra Mussel substrate monitor was inspected on August 5, no mussels were observed.  The next inspection for zebra mussels is scheduled for September 1.

Avian Activity:  Up to 15 gulls and 16 cormorants were observed in the vicinity of the dam during this report period.

Project:  Lower Granite

# Biologists:  Mike Halter and Ches Brooks

Dates:  August 20 - 26, 2010

## Turbine Operation

Lower Granite had turbine units 1, 2, 5 and 6 available for power generation at the beginning of the report week.   Turbine unit 3 remained out of service for a rewind and will not be available until approximately December 31, 2010.  Turbine unit 4 remained out of service for annual maintenance.  The estimated return to service date is September 3.  Summer spill operations began on June 21 and are planned to continue through August 31, at the rate of 18.6 kcfs using the spill plan specified in the 2010 Fish Passage Plan.

## Adult Fish Passage Facility

On August 20-22 the Lower Granite fisheries biologists performed measured inspections of the adult fishway system.

Fish Ladder:  All criteria were met during the inspections this week.

Fishway Entrances and Collection Channel:   Head differential readings remained within criteria at all adult fishway entrances this week.  Weir depths at the south shore fishway entrances were on sill on the August 20 and 22 inspections and met criteria on the August 21 inspection.  South shore entrance weir depths ranged from 7.2 to 8.1 feet this week.  North powerhouse fishway entrance weirs were on sill all week due to tailwater elevations below 636.0 feet.  Weir depths at the north powerhouse entrances ranged from 4.5 to 5.2 feet.  Weir depths at the north shore fishway entrances were out of criteria all week with readings of between 4.9 and 5.1 feet (criterion ≥ 7.0 feet).  Weir depth readings at the north shore are being sacrificed in order to maintain the requisite 1.0 foot of head differential.  Weekly velocity readings in the adult fishway collection channel transition pool area ranged from 0.89 to 1.12 feet per second and averaged 0.99 feet per second.

### Auxiliary Water Supply System:  Fish pumps 2 and 3 were operated during the week without any problems.

#### Juvenile Fish Passage Facility

The juvenile fish sample rate remained at the 100% level all week.

Forebay Debris/Gatewell Debris/Oil:  Debris levels were low during the week.  It was not necessary to hand-dip debris from any gatewells this week.   A slight film was noted on the surface of bulkhead slot 4C.  Oil soaker pads were put in place to help remove the film.

ESBSs/VBSs:   The last video inspections were conducted on June 26 and 27.  No problems were detected.   The next video inspections are scheduled for August 28 through 29.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:   Debris levels remained very light during the week.  Collection gallery orifices were back-flushed every three hours around the clock to ensure they remained free of materials that might impact fish passage.

Transportation Facility:   The facility operated in a 100% sampling mode with all collected fish being diverted to the sample holding tank outside the lab.  Fish are being worked up every other day on designated transport dates.  All facets of facility operation went smoothly this week.

Transport Summary:  Fish are being trucked on the even days of the month.  Fish numbers remained low enough to utilize the pickup midi-tanker (300 gallon tank) on all trips this week.  Fish are being released into the outfall pipe below Bonneville Dam.

Removable Spillway Weir:   Nothing new to report on the RSW.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Granite Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 31.0 | 26.3 | 18.2 | 13.4 | 66.1 | 66.0 | 5.0 | 5.0 |

*Cooling line intake temperature.

Other

Inline Cooling Water Strainers:  Inspection of the inline cooling water strainers took place on August 5.   No lamprey were removed from the strainers on this inspection.   The next inspections will take place in mid to late September.

Invasive Species:   The zebra mussel trap at the adult fishway exit was examined for zebra mussels on the Aug 22 inspection.  No evidence of zebra mussels was found.

Research: The adult fish trap was closed at 0900 hours on August 13 due to water temperatures in excess of 70° degrees F.   The trap was watered up August 21 after water temperatures dropped below the 70°F threshold.  Normal adult fish sampling operations began the following day at 1105 hours.  The sample rate was set at 12% this week but may very well change depending on fish numbers.  One out of every 20 hatchery steelhead and all wild PIT-tagged steelhead are being scale-sampled.  Scale samples and genetic samples are being taken from hatchery PIT-tagged steelhead.  Transport of fall Chinook to Lyon’s Ferry Hatchery and the Nez Perce Hatchery at Cherry Lane will begin as soon as fish numbers warrant.  Seventy percent of the transported fish are to be transported to Lyon’s Ferry and the remaining 30% are to go to Cherry Lane.  The Nez Perce Tribe will haul fish on Sundays and Mondays and Lyon’s Ferry will haul fish Tuesday through Saturday.