U.S. ARMY CORPS OF ENGINEERS

WALLA WALLA DISTRICT

#### FISH FACILITIES WEEKLY REPORT

#32-2010

# Project:  McNary

# Biologists: Bobby Johnson

Dates:  October 1 - 7, 2010

## Turbine Operation

McNary had nine to ten turbine units available for power generation this week.  Turbine units 2 and 7 remained out of service for rewind.  Turbine unit 1 remains out of service in support of T1 transformer replacement which will take approximately three weeks.  Other outages taking place this week included turbine unit 8 from October 4 to 7 (T4 transformer maintenance),   turbine units 6 and 13 on October 5 ( ESBS camera inspections, 64 minutes total outage time), and turbine unit 6 on October 5 and 6 (failed ESBS in slot 6A).  All available turbine units were operated within the 1% best efficiency guidelines this week.

## Adult Fish Passage Facilities

On October 2, 4 and 7, the McNary fisheries biologist performed measured inspections of the adult fishways.  Adult fish counts continued.  Oregon entrance weirs SFEW1, SFEW2 and NFEW1 are being lowered on scheduled nights as part of the adult lamprey passage study.  No problems have been observed. Technicians are now available to monitor the weirs while the juvenile facility is in fall primary bypass season.

Fish Ladders:  Both ladders met all Fish Passage Plan criteria during measured inspections.  On October 3, the Washington ladder regulating weir triggered an alarm three times.  The Oregon ladder regulating weir also triggered an alarm the same day.  In each case, normal operation was restored after the alarm was reset. The picketed leads at the Oregon count station were cleaned this week as they became obstructed with Eurasian milfoil.  The north traveling screen at the Oregon exit remains out of service.   The south Oregon exit traveling screen operation and differential monitoring results have been satisfactory.  Both exits are being monitored daily as Eurasian milfoil continues to be a problem.

Fishway Entrances and Collection Channel:  All Washington ladder inspection points were in criteria this week as were most Oregon ladder inspection points.  On October 2 and 4, the north powerhouse pool differential respectively measured 0.9 and 0.7 feet.  All other Oregon ladder inspection points met criteria.  Collection channel velocities averaged 1.3 feet per second.

### Auxiliary Water Supply System:  For the week, fish pumps 1 and 2 ran with blade angles of 30 degrees.  Only pumps 1 and 2 are being operated due to the T1 transformer outage.  The juvenile facility supplied the usual 450 cfs to the north powerhouse pool and the Wasco County PUD turbine unit had no interruptions in service this week

### Juvenile Fish Passage Facility

Collection for fish transport ended on October 1 at 0700 hours when the facility was switched to primary bypass operations.  No other changes in facility operations occurred.  For report day of October 1 (and for the week), 130 smolts, no juvenile lamprey, and 30,985 juvenile shad were collected and transported.

Forebay Debris/Gate well Debris/Oil:  Forebay debris was light and consisted of mostly Eurasian milfoil.  Trash rack differential measurements were satisfactory and no racks were cleaned.  Slots 1B, 1C, 2A, and 2B remained covered at the beginning of this report period in support of transformer T2 and T7 replacements.  Slots 14A, 14B and 14C were covered on October 4 in support of forebay intake deck (crane 5) maintenance.  No problems were observed in the gatewell slots.

ESBSs/VBSs:  ESBSs are currently deployed in all turbine units.  The screens in slots 5C, 6A, 9C and 10C remain in transducer bypass mode.  ESBSs video camera inspections were conducted on October 5 in turbine units 6 and 13.  The ESBS cleaning brush in slot 6A appeared to be inoperative even though the screen was in transducer bypass mode.  Turbine unit 6 was declared out of service and another screen was installed the following day.  The new screen subsequently triggered an alarm and was placed in manual operation.  Electrical crews were able to resolve the problem and restore the screen to normal operation.  All other ESBSs operated satisfactorily.  VBS monitoring revealed four screens out of criteria.  These screens and eleven others were cleaned on October 6 and 7.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:  For the week, 39 orifices were in use as forebay elevations were stable.  An orifice associated with turbine unit 2 (currently out of service) continues to be opened or closed as required.  Reduced orifice count during high forebay elevations allows more efficient operation of the side dewatering valves.  All systems operated well in automatic mode.  Technicians have been monitoring the collection channel 24 hours per day since the end of the fish transport season.  This monitoring continues during VBS cleaning operations.

Transportation Facility:  On October 1 at 0700 hours, truck transportation concluded with the beginning of fall primary bypass season.  All PIT tag and sample gates were turned off for the year.  All fish were evacuated to the raceway and loaded in to the fish trailer for the final trip to the release site below Bonneville Dam.  The system which will remain watered up to avoid possible freeze breakage which may occur prior to the start of winter maintenance in December.  Until then, only minor maintenance will be performed at the collection facility.

Transport Summary:  The last truck of the season departed the McNary facility on October 1.  The trip was completed without incident.  Winter fish trailer maintenance will soon begin.

River Conditions

River conditions during the week are outlined in Table 1 as provided by the control room as the smolt monitoring staff is no longer collecting data.  The control room data day runs from 0000 to 0000 hours each day.

Table 1.  River conditions at McNary Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 113.0 | 78.0 | 0.0 | 0.0 | 66 | 65 | 6.0 | 6.0 |

Other

Inline Cooling Water Strainers:  Cooling water strainer inspections last took place on August 18 in turbine unit 6.  No salmonids, lamprey or other fish were noted.

Invasive Species: No invasive species were found during the September 30 detector inspection.  The next inspection is scheduled to take place November 1.

Predatory bird counts concluded on October 1.  Casual tail water observations noted the presence of gulls, Forester’s terns, mergansers and cormorants.  Most tailwater birds were seen roosting in the spill basin or feeding in the powerhouse flow.  Bird numbers appear to fluctuate with the juvenile shad outmigration.  An occasional cormorant, gull or tern was seen around the bypass outfalls.  A small group of grebes was occasionally observed in the forebay along with and occasional gull or cormorant.  No grebes were seen in the gatewell slots.  The three grebes noted in the collection channel at the end of the previous report period are still present.  The water cannon continued to operate satisfactory.

Project:  Ice Harbor

# Biologist: Mark Plummer

Dates:  October 1 - 7, 2010

## Turbine Operation

All turbine units were available for operation during this reporting period.  Turbine unit 3 went out of service for overhaul October 4 at 0645 hours.

## Adult Fish Passage Facilities

### Fish facility personnel inspected the adult fishways on October 1, 4, 5, 6, and 7.

### Fish Ladders:  All north and south fish ladder inspection areas (picketed leads, head differentials, fishway exits, and depth over weirs) were within criteria.  Head over the weir criteria is 1.0 – 1.3 feet.

### Fishway Entrances and Collection Channel (inspection date order):  The south shore entrance (SFE) was on sill with a depth of 8.7 feet, on sill with a depth of 6.7 feet, off sill with a depth of 7.5 feet, off sill with a depth of 7.3 feet, and off sill with a depth of 7.3 feet.  The north powerhouse entrance (NFE) was off sill with a depth of 7.6 feet, on sill with a depth of 6.7 feet, off sill with a depth of 7.8 feet, off sill with a depth of 7.8 feet, and on sill with a depth of 7.5 feet.  The north shore entrance (NSE) was on sill with a depth of 7.2 feet, on sill with a depth of 6.7 feet, off sill with a depth of 7.4 feet, on sill with a depth of 7.6 feet, and on sill with a depth of 7.6 feet.  Fishway entrance criterion is 8 feet depth, greater than 8 feet depth, or on sill.  All out of criteria depths were reported to the operator for adjustments.  All channel/tailwater differentials were in criteria.  Visual staff gage or measuring tape readings are reported in this report.

### Auxiliary Water Supply System:  Two of the 3 north shore fish pumps were operated without problems.  Five of 8 south shore fish pumps were operated without problems.  South shore fish pump 1 remains out of service due to vibration. South fish pump motor replacements have now been completed.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  There is minimal forebay debris.  Fish ladder exits are clear of debris and the bubblers are operating satisfactorily.

STSs/VBSs:  STSs operations have been in cycle run mode as since August 30.  No problems were found during the September STS/VBS inspections.  October STS/VBS inspections are scheduled for the 18th and 20th.

Orifices, Collection Channel, Dewatering Structure, and Bypass Pipe:   The juvenile fish bypass is in operation.  20 orifices are open. The dewatering structure weirs were returned to automatic operation October 4.

Juvenile Bypass Facility:  The bypass remains in service and is operating satisfactorily.

Juvenile Fish Sampling:  Sampling for the 2010 season concluded July 1.

Removable Spillway Weir:  The RSW is not in operation as spill for fish passage ended September 1.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Ice Harbor Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 25.7 | 15.2 | 0.0 | 0.0 | 64 | 63 | 7.0 | 5.4 |

*Unit 1 scrollcase temperature.

Other

Inline Cooling Water Strainers:  Cooling water strainer cleanings took place in turbine units 2, 3, 4, 5 and 6 on September 13 and in turbine unit 1on September 15.  No lampreys were recovered.  However, several juvenile shad were found in the strainers.

Invasive Species:  No zebra mussels were observed during inspections performed on October 4, 5, and 7.

No adult salmonid mortalities were observed this reporting period.

# Project:  Lower Monumental Dam

# Biologist(s):  William Spurgeon and Elizabeth Lindsey

Dates:  October 1 - 7, 2010

## Turbine Operation

All turbine units were operated within hard constraint 1% operational criteria this week.  However, turbine unit 4 failed to stop on command and operated outside the 1% efficiency criteria on October 5 from 0944 to1048 hours.  Through most of this period the tubine unit 4 was essentially in speed no load mode.   Turbine unit 1 was removed from service for annual maintenance on September 8 at 1015 hours.

Adult Fish Passage Facility

The adult fishway was inspected by Corps or State biologists on October 2, 4, 5 and 6.

Fish Ladders: Fishway exit head differential and depth over the weirs were within criteria (< 0.5’ and 1.0’-1.3’, respectively) on all inspections.  Picketed lead head differentials were in criteria (< 0.4’ and < 0.3’ for north and south shore fishways, respectively) on all inspections.

### Fishway Entrances and Collection Channel:  NPE 1 and NPE 2 weir gates were in depth criteria (criteria: > 8’ or on sill) on all inspections.  North shore channel/tailwater head was in criteria (1’-2’) on all inspections.

SPE 1 and SPE 2 weir gates were in sill criteria (criteria: > 8’ or on sill) on all inspections.  While on sill, the gate depth readings were 7.5’, 7.4’, 7.5’, and 6.8 feet.  South powerhouse channel/tailwater head was in criteria (1’-2’).

SSE1 weir gate was in depth criteria (criteria: > 8’ or on sill) on all inspections.   SSE2 was in criteria (6’ above sill) on all inspections.  South shore channel/tailwater head was in criteria (1’-2’).

### Auxiliary Water Supply System: AWS pump 3 remains out of service to replace the bearing housing and repair the diffuser assembly.  Pumps 1 and 2 were in service throughout this period.  Pump 3 remains “bulkheaded off” to minimize water loss during two-pump operation.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  There was an average of 2.1 square yards of forebay debris observed during this period.  Gatewell debris surface coverage ranged from 0-30 percent.  No oil was observed in gatewells.

STSs/VBSs:  STS operation was changed to cycle mode on August 10 when the average fork length of sub-yearling Chinook and/or sockeye exceeded 120mm.  STS inspections were conducted October 4 and 5.  All screens passed inspection.

Orifices, Collection Channel, Dewatering Structure, Flume:  The collection channel is operating with 18 orifices open.

Collection Facility:  The primary dewaterer mechanical screen cleaner was removed from service on August 31 due to travel cable slippage.  The pneumatic screen cleaner (bubbler) is more than adequate for keeping the screen clean through the remainder of the season.  Facility operation changed to primary bypass on October 1.  The load and hold facility is now in winter maintenance mode.

Transport Summary:  Transport ended with the October 1 fish truck departure.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Monumental Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature (oF)* |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 26.8 | 15.4 | 0.0 | 0.0 | 65.0 | 63.5 | 4.9 | 4.0 |

*Scrollcase temperatures.

#### Other

Inline Cooling Water Strainers:  Turbine unit cooling water strainers were cleaned on October 5.  Results are outlined in Table 2 below.

|  | Lamprey |  | Salmon |  | Steelhead |  | Siberian Prawn |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | Live | Dead | Live | Dead | Live | Dead |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 0 | 0 | 2 |
| 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

Invasive Species:  There were no zebra mussels observed at the monitoring stations on September 10.

Project:  Little Goose

# Biologist(s):  Richard Weis

Dates:  October 1 - 7, 2010

## Turbine Operation

Turbine units 1, 2 and 4 through 6 were available for power generation throughout this reporting period.  Turbine unit 3 was removed from service on September 27 at 0650 hours for annual maintenance.  There were no reports of turbine units operating outside 1% of best efficiency.

## Adult Fish Passage Facility

ODFW and COE Fisheries Biologists performed measured inspections of the adult fishway on October 1, 5 and 6.

Fish Ladder:  The ladder exit head differential remained steady at 0.1 feet (criteria < 0.5 ft.).  Water depths over the weirs remained at 1.2 feet (criteria 1.0-1.3 ft.) and picketed lead head differentials ranged between 0.0 and 0.1 feet (criteria < 0.3 ft.).

Fishway Entrances and Collection Channel:  Channel to tailwater head differentials at fish entrances ranged between 0.96 and 1.64 feet (criteria 1.0 to 2.0 ft.).  SSE weir depths ranged between 8.30 and 8.36 feet (criteria ≥8.0 ft),   NPE weir depths ranged between 7.04 and 7.76 feet (criteria ≥7.0 ft. or on sill).  NSE1 is manually positioned at 529.1’ and depths measured between 7.38 and 10.44 feet (criteria ≥ 6.0 ft.).  NSE2 weir depth ranged between 6.14 and 6.18 feet (criteria ≥ 6.0 ft.).

Collection channel surface water velocities (criteria 1.5 to 4.0 fps) ranged between 2.2 and 2.4 fps near the SSE and 1.9 and 2.7 fps near the NSE.   Subsurface water velocities near the SSE (flowmeter) ranged between 1.2 and 1.6 fps.

Auxiliary Water Supply System:  All fish pumps operated within criteria during this report period.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  Surface woody debris in the immediate forebay ranged between 100 to 600 square feet.   Gatewells remain clear of debris.

Spillway Weir:  The weir was removed from service on August 5.

ESBS/VBS:   All operating ESBSs were within criteria during this report period, except for the fishscreen in slot 2C which was found off line on Oct. 7.  Normal operation resumed after the circuit breaker was reset.

Orifices, Collection Channel, Dewatering Structure, Flume:  The number of open orifices in the juvenile collection channel were reduced from 21 to 20 during this report period in response to increased pool elevations.  No problems to report.

Transportation Facility:  Collection and transport continued through this report period.  Daily collection for the period ranged between 32 and 1048 for a total of 2,532 for the week.  All fish were transported except for mortalities.  Descaling and mortality rates for this report period are 0.7% and 1.9% respectively.

Transport Summary:  Every other day truck transport operations continued this report period.  Salt in small concentrations of approximately 1 g/L is added to the midi-tank water to treat potential Columnaris disease and reduce stress.  “Piggybacking” with the Lower Granite Juvenile Fish Facility occurred on Oct. 3 and 5 due to increases in fish numbers.  All transport operations were successfully performed.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Little Goose Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 26.6 | 14.8 | 0 | 0 | 65.5 | 64.3 | 6.0+ | 6.0 |

*Ladder temperature.

Other

Inline Cooling Water Strainers:  Cooling Water Strainers were not inspected during this report period.

Invasive Species:  The zebra mussel substrate monitor was inspected on October 6, no mussels were observed.  The next inspection for zebra mussels is scheduled for November 3.

Avian Activity:  Daily number of piscivorous birds observed in the vicinity of the dam during this report period included up to 20 gulls and 25 cormorants.

Project:  Lower Granite

# Biologist(s):  Mike Halter and Ches Brooks

Dates:  October 1 - 7, 2010

## Turbine Operation

Lower Granite had turbine units 1, 2, 5 and 6 available for power generation at the beginning of the report week.   Turbine unit 3 remained out of service for a rewind and will not be available until approximately December 31, 2010.  Turbine unit 4 remained out of service for annual maintenance with an estimated return to service date of October 15.  Turbine unit 2 was taken out of service for annual maintenance at 1300 hours on October 4.  The return to service date is October 22.

## Adult Fish Passage Facility

On October 1, 3 and 4, the Lower Granite fisheries biologists performed measured inspections of the adult fishway system.

Fish Ladder:   All criteria were met during the report period.

Fishway Entrances and Collection Channel:   Head differential readings remained within criteria at all adult fishway entrances this week.  Weir depths at the south shore fishway entrances were also within criteria on all inspections this week.  North powerhouse fishway entrance weirs were on sill during the October 3 inspection due to tailwater elevations below 636.0 feet.  North powerhouse entrances met the desired criteria level on the October 1 and 4 inspections as tailwater elevation exceeded 636.0 feet. Weir depths at the north powerhouse entrances ranged from 7.9 to 8.2 feet during the week.   Weir depths at the north shore fishway entrances were out of criteria all week with readings of between 4.9 and 5.0 feet (criterion ≥ 7.0 feet).  Weir depth readings at the north shore are being sacrificed in order to maintain the requisite 1.0 foot of head differential.  Weekly velocity readings in the adult fishway collection channel transition pool area ranged from 0.83 to 1.14 feet per second and averaged 0.98 feet per second.

### Auxiliary Water Supply System:  Fish pumps 2 and 3 were operated during the week without any problems.

#### Juvenile Fish Passage Facility

Subyearling chinook numbers increased during the week and the sample rate was decreased from 100% to 50% at 0700 hours on October 4.  The sample rate was returned to 100% at 0700 hours on October 7 but then reduced back to 50% at 1000 hours on October 7 due to a sudden increase in fish numbers.

Forebay Debris/Gatewell Debris/Oil:  Debris levels were low during the week.  It was not necessary to hand-dip debris from any gatewells.

ESBSs/VBSs:   The last ESBS/VBS video inspections took place on August 28-29.  No new problems were found.  The next inspections are scheduled for October 21-22.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:   Debris levels were very light during the week.  Collection gallery orifices were backflushed every three hours around the clock to ensure they remained free of materials that might impact fish passage.

Transportation Facility:   The facility operated in 100% sampling mode with all collected fish being diverted to the sample holding tank outside the JFF lab until increased fish numbers necessitated the use of a raceway.  Fish are being worked up every other day on designated transport dates.  All facets of facility operation continue to go smoothly.  An increase in fish numbers and marking operations by NOAA-Fisheries did impact the truck departure on October 5.  The truck did not leave Lower Granite until 1130 hours and then had to stop at Little Goose to pick up additional fish due to an increase in fish numbers there also.

Transport Summary:  Fish are being trucked on the odd days of the month.  Fish numbers were low enough to utilize the pickup midi-tanker (300 gallon tank) on the October 1 trip this week.  Due to increased fish numbers and the need to also pick up the fish collected for transport at Little Goose, the semi truck and trailer was used for the October 3 and 5 trips.  On October 7, fish numbers were again low enough to utilize the pickup midi-tanker.  Fish are being released into the outfall pipe below Bonneville Dam.  Transport operations have been going well.

Removable Spillway Weir:   Nothing new to report on the RSW.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Granite Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 26.5 | 17.1 | 0.0 | 0.0 | 67.0 | 66.2 | 4.9 | 4.4 |

*Cooling line intake temperature.

Other

Inline Cooling Water Strainers:  Cooling line strainer inspections took place on September 16.  Mortalities recovered included one juvenile lamprey and one juvenile salmonid.

Invasive Species:  The zebra mussel trap at the adult fishway exit was examined for zebra mussels on October 3.  No evidence of zebra mussels was found.

Research:  The sample rate at the adult fish trap remained at 10% during the report week.  The protocol for taking scale samples from hatchery steelhead remained at one in 20 fish.  (These protocols will probably change again next week.)  PIT-tagged Chinook are still being released back into the ladder following an operculum punch.  Transport of adult fall Chinook to Lyon’s Ferry Hatchery continued during the week.  Fish are being transported to Lyon’s Ferry when numbers warrant (approximately one load every other day).  The Nez Perce Hatchery at Cherry Lane has met their quota for the year and no longer is transporting fall Chinook.

NOAA-Fisheries personnel continued PIT-tagging subyearling Chinook in the juvenile fish facility lab to evaluate late season truck transport operations at Lower Granite.