U.S. ARMY CORPS OF ENGINEERS

WALLA WALLA DISTRICT

#### FISH FACILITIES WEEKLY REPORT

#33-2010

# Project:  McNary

# Biologists: Bobby Johnson

Dates:  October 8 - 14, 2010

## Turbine Operation

McNary had eleven units available for power generation this week.  Units 2 and 7 remain out of service for a rewind contract until year’s end.  Unit 1 remains out of service for transformer T1 replacement contract which will be completed in approximately two weeks.  On October 12, unit 4 was out of service for 28 minutes for an ESBS camera inspection.  The hard one percent criterion continues with no units running outside the criteria this week.

## Adult Fish Passage Facilities

On October 9, 12 and 14, the McNary fisheries biologist performed measured inspections of the adult fishways.  Adult fish counting season continues.  Similar to last year, lowering Oregon entrance weirs SFEW1, SFEW2 and NFEW1 on scheduled nights continues as part of the adult lamprey passage study with no problems observed as technicians are now available to monitor the weirs while the juvenile fish facility is in primary bypass for the fall season.

Fish Ladders:  During measured inspections, all Fish Passage Plan criteria were met on both ladders.  On October 14, the Washington ladder’s regulating weir and tilting weir 340 alarmed and were reset.  Also, on October 14, the Oregon exit’s crane was found to have structural issues which required it to be removed from service.  The picketed leads will now be cleaned with a portable crane.  The Oregon exit’s the north traveling screen remains out of service.  The south traveling screen has been operating well and differential monitoring has revealed no problems.  Both exits are being monitored daily as milfoil continues to be a problem.

Fishway Entrances and Collection Channel:  At the Washington ladder, all inspection points were in criteria.  On October 9, the Oregon ladder’s south powerhouse entrance, SFEW2, measured 8.8 feet.  Also, on October 12 and 14, SFEW1 measured 8.8 feet.  All other Oregon ladder inspection points were in criterion.  The collection channel velocity average 1.4 feet per second.

Auxiliary Water Supply System:  For the week, fish pumps 1 and 2 ran with blade angles of 30 degrees with no interruptions.  During the transformer T1 outage, pumps 1 and 2 will be in service.  Also, the juvenile facility supplied the usual 450 cfs to the north powerhouse pool with no interruptions.  Finally, the Wasco County PUD unit also had no interruptions in service.

### Juvenile Fish Passage Facility

Fall primary bypass season continues with PIT tag detect occurring in the full flow flume.  No changes in facility operations occurred.  The system will remain watered up to avoid possible freeze breakage which may occur before winter maintenance season begins in December.

Forebay Debris/Gate well Debris/Oil:  Forebay debris was light and consisted of mostly Eurasian milfoil and woody debris.  Trash rack differentials revealed no problems and no racks were cleaned.  For the transformer contracts, 1B, 1C, 2A, and 2B slots remained covered.  For the maintenance contract on the forebay deck crane, the slots at 14A, 14B and 14C also remain covered.  No problems were observed in the gatewell slots.

ESBSs/VBSs:  ESBS’s are installed at all units.  The screens at 5A, 5C, 9C and 10C slots remain in transducer bypass mode.  The ESBS camera inspections at unit 4 revealed no problems.  VBS monitoring revealed no screens out of criteria.  On October 14, four screens were cleaned as a preventative measure.  With the deck crane out of service, VBS’s will be cleaned with a portable crane if required.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:  For the week, 39 to 40 orifices were in use depending on forebay elevation.  The orifices at unit 2 which is out of service continue to be opened or closed as required as a reduced orifice count during high forebay elevations allows the side dewatering valves to operate in a more favorable range.  On October 13, the orifice at 14C slot was found closed.  Assuming the technician on the previous shift had forgotten to reopen it, the orifice was closed for less than eight hours.  The biologist reopened the orifice and reviewed channel procedures with all technicians.  No harm to fish was noted.  All systems operated well in automatic mode.  The channel was monitored during VBS cleaning and on all shifts during the fall season.

Transportation Facility:  The facility remains shut down for fall primary bypass season.  All systems are off for the year.  Little maintenance along with safety modifications will be preformed at the facility until winter season begins.  On October 14, an unscheduled 20 minute power outage occurred with no adverse affect.

Transport Summary:  There is nothing to report.  Winter maintenance on the fish trailers continues.

River Conditions

River conditions during the week are outlined in Table 1 as provide by the control room as the smolt monitoring staff is no longer collecting data.  The control room data day runs from 0000 to 0000 each day.

Table 1.  River conditions at McNary Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 133.6 | 91.5 | 0.0 | 0.0 | 65 | 63 | 6.0 | 6.0 |

Other

Inline Cooling Water Strainers:  Cooling water strainer inspections last took place on August 18 in turbine unit 6.  No salmonids, lamprey or other fish were noted.

Invasive Species: The next the zebra mussel station examination will occur in early November.

On October 1, predatory bird counts concluded.  Casual observations of the tail water area revealed gulls, mergansers and cormorants.  Most birds were seen roosting near the spill basin or feeding in the powerhouse flow.  Bird numbers appear to be fluctuating with juvenile shad out migration.  At the bypasses’ out falls, an occasional merganser, cormorant and gull were noted.  The water cannon operated well.  In the forebay area, groups of grebes were occasionally noted along with the occasional gull and cormorant.  No grebes were observed in the gatewell slots.  One of the three grebes in the collection channel passed out of the system leaving two.

Project:  Ice Harbor

# Biologist: Mark Plummer

Dates:  October 8 - 14, 2010

## Turbine Operation

Turbine units 1, 2, and 4 - 6 were available for operation this reporting period.  Turbine unit 3 remained out of service for overhaul.

## Adult Fish Passage Facilities

### Fish facility personnel inspected the adult fishways October 8, 12, 13, and 14.

### Fish Ladders:  All north and south fish ladder inspection areas (picketed leads, head differentials, fishway exits, and depth over weirs) were within criteria. Over the weir criteria is 1.0 – 1.3 feet.

### Fishway Entrances and Collection Channel (inspection date order):  The south shore entrance (SFE) was off sill with a depth of 7.4 feet, off sill with a depth of 7.4 feet, on sill with a depth of 6.7 feet, and on sill with a depth of 6.9 feet.  The north powerhouse entrance (NFE) was off sill with a depth of 8.0 feet, off sill with a depth of 7.9 feet, on sill with a depth of 6.7 feet, and on sill with a depth of 6.9 feet.  The north shore entrance (NSE) was off sill with a depth of 8.0 feet, on sill with a depth of 7.7 feet, off sill with a depth of 7.2 feet, and off sill with a depth of 7.8 feet.  Fishway entrance criterion is 8 feet depth, greater than 8 feet depth, or on sill.  All out of criteria depths were reported to the operator to make adjustments.  All channel/tailwater differentials were in criteria.  Visual staff gage or measuring tape readings are reported in this report.

### Auxiliary Water Supply System:  2 of the 3 north shore fish pumps were operated without problems.  5 of 8 south shore fish pumps were operated without problems.  South shore fish pump 1 remains out of service due to vibration.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  There is minimal forebay debris.  Fish ladder exits are clear of debris and the bubblers are operating.

STSs/VBSs:  The STSs operation is in cycle run mode.  No problems were found during the September STS/VBS inspections.  October STS/VBS inspections are scheduled for the 18 and 20.

Orifices, Collection Channel, Dewatering Structure, and Bypass Pipe:   The juvenile fish bypass is in operation.  20 orifices are open. The dewatering structure weirs were returned to automatic operation October 4.

Juvenile Bypass Facility:  The bypass was put in operation March 8, 2010.

Juvenile Fish Sampling:  Sampling for the 2010 season concluded July 1 due to the number of juvenile fish sampled during a 4 hour period dropping below 100 for 2 consecutive samples.

Removable Spillway Weir:  The RSW is not in operation.  Spill for fish passage ended September 1.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Ice Harbor Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 25.7 | 15.2 | 0.0 | 0.0 | 64 | 63 | 7.0 | 5.4 |

*Unit 1 scrollcase temperature.

Other

Inline Cooling Water Strainers:  Main turbine unit cooling water strainer inspections are performed during STS/VBS inspections.  Main turbine unit 3 strainers were inspected October 14 by maintenance staff.  They reported that nothing was found.

Invasive Species: No zebra mussels were observed when the inspection was performed October 12 and 13.

The water velocity meter located in the south adult fishway is not functioning.  Repairs/replacement will be done during the adult fish winter maintenance period.

No adult salmonidae mortalities observed this reporting period.

# Project:  Lower Monumental Dam

# Biologist(s):  William Spurgeon and Elizabeth Lindsey

Dates:  October 8 - 14, 2010

## Turbine Operation

Turbine units were operated in hard constraint 1% operational criteria.  Unit 1 was removed from service for annual maintenance on September 8 at 1015 hours and returned to service on October 12 at 1127 hours.  Unit 4 was removed from service for annual maintenance on October 13 at 0714 hours.

Adult Fish Passage Facility

The adult fishway was inspected by Corps or State biologists on October 9, 13, and 14.

Fish Ladders: Fishway exit head differential and depth over the weirs were within criteria (< 0.5’ and 1.0’-1.3’, respectively) on all inspections.  Picketed lead head differentials were in criteria (< 0.4’ and < 0.3’ for north and south shore fishways, respectively) on all inspections.

### Fishway Entrances and Collection Channel: NPE 1 and NPE 2 weir gates were in depth criteria (criteria: > 8’ or on sill) on all inspections.  North shore channel/tailwater head was in criteria (1’-2’) on all inspections.

SPE 1 and SPE 2 weir gates were in sill criteria (criteria: > 8’ or on sill) on all inspections.  While on sill, the gate depth readings were 6.3’, 6.8’, and 7.3 feet.  South powerhouse channel/tailwater head was in criteria (1’-2’).

SSE1 weir gate was in depth or sill criteria (criteria: > 8’ or on sill) on all inspections.  While on sill, the gate depth readings were 7.4’ and 7.9’ feet.  SSE2 was in criteria (6’ above sill) on all inspections.  South shore channel/tailwater head was in criteria (1’-2’).

### Auxiliary Water Supply System: AWS pump 3 remains out of service to replace the bearing housing and repair the diffuser assembly.  Pumps 1 and 2 were in service throughout this period.  Pump 3 is bulkheaded off to minimize water loss during two-pump operation.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  There was an average of 18.3 square yards of forebay debris observed during this period.  Gatewell debris surface coverage ranged from 0-35 percent.  No oil was observed in gatewells.

STSs/VBSs:  STS operation was changed to cycle mode on August 10 as average fork length of sub-yearling chinook and/or sockeye exceeded 120mm.  STS inspection was conducted on October 4 and 5.  All screens passed inspection.

Orifices, Collection Channel, Dewatering Structure, Flume:  The collection channel is operating with 18 orifices open.

Collection Facility:  The primary dewaterer mechanical screen cleaner was removed from service on August 31 due to slipping of the travel cable.  The pneumatic screen cleaner is more than adequate for keeping the screen clean through the remainder of the season.  Facility operation changed to primary bypass on October 1.  The load and hold facility is now in winter maintenance mode.

Transport Summary:  Transport ended with the October 1 load.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Monumental Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature (oF)* |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 23.6 | 14.8 | 0.0 | 0.0 | 63.5 | 62.8 | 5.0+ | 4.1 |

*Scrollcase temperatures.

#### Other

Inline Cooling Water Strainers:  Turbine unit cooling water strainers were cleaned on October 5.  Results are outlined in Table 2 below.

Table 2.  Unit cooling water strainer inspection.  Aquatic Organisms recovered.

|  | Lamprey |  | Salmon |  | Steelhead |  | Siberian Prawn |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | Live | Dead | Live | Dead | Live | Dead |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 0 | 0 | 2 |
| 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

Invasive Species:  There were no zebra mussels observed at the monitoring stations on October 14.

Fish Salvage: Unit 4 scroll case was dewatered for annual maintenance on October 14.  One deceased sculpin was found.

Project:  Little Goose

# Biologist(s):  George Melanson

Dates:  October 8 - 14, 2010

## Turbine Operation

Turbine units 1, 2 and 4 through 6 were available for power generation throughout this reporting period.  Turbine unit 3, removed from service on September 27 was returned to service on October 13 at 1540 hours following completion of annual maintenance.  There were no reports of turbine units operating outside the 1% of best efficiency.

## Adult Fish Passage Facility

ODFW and COE Fisheries Biologists performed measured inspections of the adult fishway October 8, 11 and 13.

Fish Ladder:  The ladder exit head differential measured between 0.1 and 0.2 feet (criteria < 0.5 ft.).  Water depths over the weirs ranged between 1.1 and 1.3 feet (criteria 1.0-1.3 ft.) and head differentials between picketed leads held steady at 0 feet (criteria < 0.3 ft.).

Fishway Entrances and Collection Channel:  Channel to tailwater head differentials at fish entrances ranged between 0.98 and 1.64 (criteria 1.0 to 2.0 ft.).  SSE weir depths ranged between 8.30 and 8.38 feet (criteria ≥8.0 ft),   NPE weir depths ranged between 7.16 and 7.62 feet (criteria ≥7.0 ft. or on sill).  NSE1 is manually positioned at 529.1 and depths measured between 9.94 and 10.50 feet (criteria ≥ 6.0 ft.).  NSE2 weir depth ranged between 6.02 and 6.18 feet (criteria ≥ 6.0 ft.).

Collection channel surface water velocities (criteria 1.5 to 4.0 fps) ranged between 1.6 and 1.9 fps near the SSE and 2.3 and 2.4 fps near the NSE.   Subsurface water velocities near the SSE (flowmeter) ranged between 1.0 and 1.3 fps.

Auxiliary Water Supply System:  All fish pumps operated within criteria during this report period.

#### Juvenile Fish Passage Facility

Forebay Debris/Gatewell Debris/Oil:  Surface woody debris in the immediate forebay ranged between 100 to 600 square feet.   Gatewells remain clear of debris.

Spillway Weir:  The weir was removed from service on August 5.

ESBS/VBS:   All operating ESBS were within criteria this report period.

Orifices, Collection Channel, Dewatering Structure, Flume:  The collection channel operated with 20 and 21 open orifices during this report period.  No problems to report.

Transportation Facility:  Collection and transport continued this report period.  Daily collection for the period ranged between 39 and 195 for a total of 655.  All fish were transported except for mortalities.  Descaling and mortality rates for this report period are 0.9% and 5.8% respectively.

Transport Summary:  Alternated day truck transport operations continued this report period.  Salt in small concentrations of approximately 1 g/L is added to the midi-tank water to treat potential Columnaris disease and reduce stress.  All transport operations were successfully performed.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Little Goose Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 24.4 | 14.8 | 0 | 0 | 64.7 | 64.0 | 6.0+ | 6.0 |

*Ladder temperature.

Other

Inline Cooling Water Strainers:  Cooling Water Strainers were not inspected during this report period.

Invasive Species:  The Zebra Mussel substrate monitor was inspected on October 6, no mussels were observed.  The next inspection for zebra mussels is scheduled for November 3.

Avian Activity:  Daily number of piscivorous birds observed in the vicinity of the dam during this report period included up to 24 gulls and 6 cormorants.

Project:  Lower Granite

# Biologist(s):  Mike Halter and Ches Brooks

Dates:  October 8 - 14, 2010

## Turbine Operation

Lower Granite had turbine units 1, 5 and 6 available for power generation at the beginning of the report week.   Turbine unit 3 remained out of service for a rewind and will not be available until approximately December 31, 2010.  Turbine unit 4 remained out of service for annual maintenance.  The estimated return to service date is now October 15.  Turbine unit 2 was taken out of service for annual maintenance at 1300 hours on October 4.  The return to service date is October 22.

## Adult Fish Passage Facility

On October 8-11, the Lower Granite fisheries biologists performed measured inspections of the adult fishway system.

Fish Ladder:   All criteria were met during the report period.

Fishway Entrances and Collection Channel:   Head differential readings remained within criteria at all adult fishway entrances during the weekly inspections.  Weir depths at the south shore and north powerhouse fishway entrances were also within criteria on all inspections this week due to the tailwater elevation exceeding 636.0 feet.  Weir depths at the north shore fishway entrances were out of criteria all week with readings of between 4.9 and 5.2 feet (criterion ≥ 7.0 feet).  Weir depth readings at the north shore entrances are being sacrificed in order to maintain the requisite 1.0 foot of head differential.  Weekly velocity readings in the adult fishway collection channel transition pool area ranged from 0.79 to 1.09 feet per second and averaged 0.93 feet per second.

### Auxiliary Water Supply System:  Fish pumps 2 and 3 were operated during the week without any problems.

#### Juvenile Fish Passage Facility

Subyearling chinook numbers decreased during the week and the sample rate was increased from 50% to 100% at 0700 hours on October 13.

Forebay Debris/Gatewell Debris/Oil:  Debris levels were low during the week.  It was not necessary to hand-dip debris from any gatewells.

ESBSs/VBSs:   The last ESBS/VBS video inspections took place on August 28-29.  No new problems were found.  The next inspections are scheduled for October 21-22.

Orifices, Collection Channel, Dewatering Structure, Bypass Pipe:   Debris levels were very light during the week.  Collection gallery orifices were backflushed every three hours around the clock to ensure they remained free of materials that might impact fish passage.

Transportation Facility:   The facility operated in a 50% sampling mode; until decreased fish numbers allowed all fish to be diverted to the sample holding tank outside the JFF lab.  Fish are being worked up every other day on designated transport dates.  All facets of facility operation continue to go smoothly.

Transport Summary:  Fish are being trucked on the odd days of the month.  Fish numbers necessitated the use of the semi truck and trailer for the October 9 trip.  On October 11-13, fish numbers were again low enough to utilize the pickup midi-tanker (300 gallon tank).  Fish are being released into the outfall pipe below Bonneville Dam.  Transport operations have been going well.

Removable Spillway Weir:   Nothing new to report on the RSW.

River Conditions

River conditions during the week are outlined in Table 1.

Table 1.  River conditions at Lower Granite Dam.

| Daily Average River Flow (kcfs) |  | Daily Average Spill (kcfs) |  | Water Temperature* (oF) |  | Water Clarity (Secchi disk - feet) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| High | Low | High | Low | High | Low | High | Low |
| 26.0 | 16.0 | 0.0 | 0.0 | 66.0 | 65.1 | 5+ | 4.6 |

*Cooling line intake temperature.

Invasive Species:   The zebra mussel trap at the adult fishway exit was examined for zebra mussels on the October 3 inspection.  No evidence of zebra mussels was found.

Inline Cooling Water Strainers:  Inspection of the cooling line strainers took place on October 14.  Two lamprey morts were noted.

Research:  The sample rate at the adult fish trap remained at 10% during the report week.  In addition, the protocol for taking scale samples from hatchery steelhead remained at one in 20 fish.  PIT-tagged Chinook are still being released back into the ladder following an operculum punch.  Transport of adult fall Chinook to Lyon’s Ferry Hatchery continued during the week.  Fish are being transported to Lyon’s Ferry when numbers warrant (approximately one load every other day).  The Nez Perce Hatchery at Cherry Lane has met their quota for the year and no longer is transporting fall Chinook.

NOAA-Fisheries personnel continued PIT-tagging subyearling Chinook in the juvenile fish facility lab to evaluate late season truck transport operations at Lower Granite.